===========================================================================

LYNX INTERFACE FOR MGD2 DOCTOR SYSTEM- HOW TO DO IT!

---------------------------------------------------------------------------

Written & (C) 1990,92  By -= TheEntity =-

Send bug reports to: Dark Tower BBS -- 801-476-9068 -> TheEntity

MAXXED OUT!    -- 503-752-6299 -> /X\.BiSON

===========================================================================

:::::::::::::::::

:::VERSION 1.0:::

:::::::::::::::::

TECHNICAL OVERVIEW OF LYNX CARTRIDGE/ROM STORAGE:

-------------------------------------------------

* Lynx cards contain game program data on either MASK ROM and/or OTP ROM.

The Lynx video card is used more like a DISK STORAGE device rather than

a DMA type of device.

* This information is based on my own reverse engineering of a LYNX I,

also known as the 'LYNX CLASSIC'.  The lynx has enough addressing space

for 512K bytes (4 MBIT), but technically, there should be 2 cartridge

port strobes available, so that is 512K X 2 = 1 megabyte (8 MBIT).

* The Lynx game system cartridge data bus is 8-bits wide, therefore all

rom cards are 8-bits.  The access time available at the pins of the rom

should be at least 250-350ns.  Since most (maybe all) of the MGD2 8M

doctor cards (rampacks) should have emulation SRAM at least as fast as

100ns, no problems should arise with getting data from the doctor card

when needed during a read-cycle.

* Current Lynx game cards are all 1Mbit and 2Mbit in size.  Battery back

RAM is possible in a lynx card, but would add extra cost to the Lynx

game card it would be used in.  If battery back RAM is incorporated into

Lynx game cards, the doctor-interface design must accomodate this extra

RAM-- read 'Part II' below for more information on this.

---

Part I-  BUILD THE INTERFACE:

-----------------------------

ITEMS NEEDED:

* MGD2/PC-ENGINE interface (GG,GB May also work)

* Lynx video game card

* Soldering/Desoldering Iron

* Wire cutters and/or hack-saw

* Razor Blade

* 30 AWG wire-wrap + wrapping tool (gun or hand-tool)

* PerfBoard, prepunched .1" centers, RadioShack special

* MultiMeter (continutity test, voltage tests)

* Ability to do fine, detailed electrical work and patience...

START:

1. Obtain PC-ENGINE interface.  Open using razor blade-- insert razor blade

into middle of pce interface.  This is where the two plastic pieces are

sealed together.  The razor blade is only use to 'cleanly' separate the

two pieces.   Afterward, if the interface does not work when you are

finished, you can opt to use razor blade on yourself  :^P

2. Remove pcb from plastic, and note the ram-pack connector that is solder

to the pcb. Remove this connector-- you can opt to attempt to desolder

it from the pcb, or you can use a wire-cutters or hack-saw to physically

cut the wire posts as close as possible to the pcb.  Do not remove the

posts from the doctor-card pcb (where the actual connector is soldered),

only remove the entire connector + assembly from the LARGER PCE interface

PCB by desoldering the wire posts on the PCE interface PCB, or cutting

the posts as close as possible to the larger PCB.  When finished, make

sure the posts are not bent and are not touching.

3. Obtain Lynx video game card.  Remove game label from top of card and

observe the mask rom pcb.  Remove this PCB from the plastic game card.

This is the pcb that the connecting 'fingers' are etched on.  After

removing, turn over and observe the mask rom or otp on reverse side.

It should contain the actual chip less ceramic or plastic DIP, with a

drop of epoxy/sealer on top.  Use razor blade and work it underneath

the chip until you cut the chip off of the board.  Do not stop and ask

why, just do it.  After removing the chip, throw it away-- you have

most likely ruined it.  Now make sure all pins are electrically isolate

from each other-- examine connections where chip was solder onto the

board and make sure everything is cleanly cut, etc..  Use razor blade

edge to scrape off any remaining wires, epoxy, etc., if needed.  If

possible, do conductivity test on all combinations of pcb pins (fingers)

to verify each pin is electrically isolate from each other. This must

be true for all pins except for #33,#31-- they should be electrically

connected.  When finished, place pcb back on lynx game card and press

firmly until it is tacked back into place.  The above is the method I

use only to create the portion of the adapter that must be plug into

the Lynx game.  A nicer method (but more complicated) would be to etch

your own PCB for use to plug into the Lynx game.  If you wish to do this,

then you need to know that the 'fingers' on the lynx game card are on

.0625 centers (1/16").  Most cheaper PCB programs for IBM or Amiga all

work around standard .025/.05/.1" cells (or multiples of that), so making

a board with cells of 1/16" centers may be a bit more tricky.

4. To make interface for following sizes of lynx game card, follow the

pinouts below, and using soldering + wire-wrapping techniques at your

disposal, connect the rampack connector to the lynx game card.

(Ex: my own working prototype is wire-wrap onto a Radio-Shack perfboard

--wire wrapped from RAMPACK CONN. to sockets, then p.t.p. wrapped to

a 34-pin connector-- 34-pin conn. plugs into modified lynx game card.)

5. PINOUTS + HOOK-UP FOR DIFFERENT SIZE LYNX GAME CARDS:

LYNX GAME PINOUTS FOR GAME CARD/CONNECTOR:

------------------------------------------

----------------    1=GND     18=A7

|                |   2=D3      19=A16

|   LYNX GAME    |   3=D2      20=A17

|      CARD      |   4=D4      21=A18

|  (LABEL SIDE)  |   5=D1      22=A19

|                |   6=D5      23=A15          * NOTE: A11 should be N.C.

|                |   7=D0      24=A14            on most or ALL Lynx I

|----------------|   8=D6      25=A13            (Classic).  It may have

|||     ||     |||   9=D7      26=A12            re-connected on Lynx II

----------------   10=En0/    27=En1/           (possibly AuxIO or En1/)

34.....17......1   11=A1      28=A8

^^^^^^^^^^^^^^^^   12=A2      29=A9

PCB FINGERS     13=A3      30=A10

(MATCH CART CONN)  14=A6      31=POWER-ON

15=A4      32=AuxIO

* POWER-ON=VCC to  16=A5      33=VCC (+5V)

turn on system   17=A0      34=SWITCH-VCC

-----------------------------------------------------------------------------

PINOUTS FOR DOCTOR-CARD/8M RAMPACK PCE CONNECTOR (GG, GB-- MAY BE SAME):

------------------------------------------------------------------------

_____________________________        1=WRITE/       21=GND

| 1- oooooooooooooooooooo -20 |       2=A15          22=A16

|21- oooooooooooooooooooo -40 |       3=A13          23=A14

_|  -------------------------  |_      4=A11          24=A12

|    |                       |    |     5=A9           25=A10

|  --     RAMPACK/PCE CONN.   --  |     6=A7           26=A8

|  |        ("TOP SIDE")       |  |     7=A5           27=A6

|  |  (SMT RESISTORS ON BOT.)  |  |     8=S-RAM/       28=A4

|  -----------------------------  |     9=A2           29=A3

|   1 ||                   || 24  |    10=A0           30=A1

------||  DOCTOR-LINK WILL ||------    11=D6           31=D7

|||   PLUG INTO THIS  |||         12=D4           32=D5

|||   PASS-THRU CONN. |||         13=D2           33=D3

|||                   |||         14=D0           34=D1

-----------------------          15=Y4/          35=Y7/

1....................25          16=Y5/          36=Y6/

17=Y3/          37=Y0/

* DON'T USE-- UNKNOWN FOR NOW         18=Y2/          38=Y1/

19=*            39=*

20=VCC (+5V)    40=*

-----------------------------------------------------------------------------

HOOKUP:

-------

** NOTE: Ln (n=# of CONNECTING PIN [1..34]) denotes LYNX GAME CARD PCB

Pn (n=# of CONNECTING PIN [1..40]) denotes PCE/RAMPACK CONN.

Dn (n=# of CONNECTING PIN [1..16]) denotes 74HC139 DECODER(DIP)

EXAMPLE OF FORMAT: Ln=Rn --> Electrically connect these two pins/wires

* USE WIRE-WRAP TECHNIQUES AND/OR SOLDER-- WHICHEVER IS APPROPRIATE TO MAKE

CONNECTION- RECOMMEND WIRE-WRAP FROM PCE/RAMPACK CONN. WIRE POSTS TO SOCKET

ON PERFBOARD *FIRST*.  CHOOSE APPROPRIATE CONNECTORS, SOCKETS, WIRE-LENGTH,

ETC., TO MAKE JOB EASIER AND MORE MODIFYABLE.

LYNX 1MBIT GAME CARD CONNECTION:           LYNX 2MBIT GAME CARD CONNECTION:

--------------------------------           --------------------------------

L1=P21    L18=P6     P1=PULLUP            L1=P21    L18=P6      P1=PULLUP

L2=P33    L19=P3     P8=PULLUP            L2=P33    L19=P23     P8=PULLUP

L3=P13    L20=P23   P15=PULLUP            L3=P13    L20=P2     P15=PULLUP

L4=P12    L21=P2    P16=PULLUP            L4=P12    L21=P22    P16=PULLUP

L5=P34    L22=P22   P17=PULLUP            L5=P34    L22=D2     P17=PULLUP

L6=P32    L23=P24   P18=PULLUP            L6=P32    L23=P3     P18=PULLUP

L7=P14    L24=P4    P19=N.C.              L7=P14    L24=P24    P19=N.C.

L8=P11    L25=P25   P35=PULLUP            L8=P11    L25=P4     P35=PULLUP

L9=P31    L26=P5    P36=PULLUP            L9=P31    L26=P25    P36=PULLUP

L10=P37    L27=N.C.  P38=PULLUP           L10=D1     L27=N.C.   P37=D4

L11=P30    L28=P26   P39=N.C.             L11=P30    L28=P26    P38=D5

L12=P9     L29=N.C.  P40=N.C.             L12=P9     L29=P5     P39=N.C.

L13=P29    L30=N.C.                       L13=P29    L30=N.C.   P40=N.C.

L14=P27    L31=L33                        L14=P27    L31=L33

L15=P28    L32=N.C.                       L15=P28    L32=N.C.

L16=P7     L33=P20                        L16=P7     L33=P20

L17=P10    L34=N.C.                       L17=P10    L34=N.C.

74HC139:                                   DECODER HOOKUP (2MBIT NEED ONLY):

--------                                   ---------------------------------

1=EnA/     9=Yb3                           D1=L10             D9=N.C.

2=A0      10=Yb2                           D2=L22            D10=N.C.

3=A1      11=Yb1                           D3=0V (GND)       D11=N.C.

4=Ya0     12=Yb0                           D4=P37            D12=N.C.

5=Ya1     13=B1                            D5=P38            D13=N.C.

6=Ya2     14=B0                            D6=N.C.           D14=N.C.

7=Ya3     15=EnB/                          D7=N.C.           D15=D16

8=GND     15=VCC(+5V)                      D8=GND,L1,P21     D16=VCC,L33,P20

---

* NOTE:  WRITE/ is used for writing data into the rampack.  Simultaneous

assertion of WRITE/ and Y[0..7]/ (overlap) will write the data

on [D0..D7] into the enabled 1MBIT SRAMS.

S-RAM/ is used for reading/writing data into the SAVE-RAM on a

MGD2 interface that incorporates 256Kbit SRAM for SAVES  (e.g. the

MD, SFC interfaces).

N.C. means NO CONNECTION (do not electrically connect to anything)

PULLUP means connect to pull-up resistor (3300 ohm is okay)

MORE SUPLEMENTS COMING SOON!.....

<EOF>

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

"KiCKiNG ASS iN tHE CoNSolE SCeNE"  -  (503) 752-6299

__________________    __  __    _________________

(_________________ \  / /  \ \  / ________________)

\ \/ /    \ \/ / ____   ___

/\  /\    /\ \  /      \  / |  __) |   \      SySoP: /X\.BiSON

/  \/  \  /  \/  \      /  \ | |__  | |\ \     Co:  RoGeR WiLCo

/ /\  /\ \/ () \/\ \    / /\ \|  __) | | ) )         Fiona

/ /  \/  \ \ ___ \ \ \  / /  \ \ |__  | |/ /          >bUg

(_/        \_)   (_) \_)(_/    \_)___) |___/

__

\/                        >HiGH SOCiETY<

____    ___   _____________             >USHQ<

/    \  /  /  /  /\__   ___/

/      \/  /  /  /   /  /

/   /\   \  \_/  /   /  /

\   \/   /      /   /  /

\______/\_____/    \_/

"NOW SUPPORTING: SNES/GENESIS/PC-ENGINE/GAME-GEAR/GAME-BOY/ & LYNX!!!"

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

----------------------- [you leeched your warez from] ------------------------

__  /|

\'o.O'     __  /|

=(___)=    \'-.O'          Sysops: ->GINGER & TCI<-

/\___U__ /\_ =(___)=____   /\____   /\____

/   ____//  / / \U/  __  \ /  __  \ /  __  \  -->-_ __   >

/   /___ /  /_/  //  /__) //  /__) //  /__) /  -  __>----_- _ _

\__    //     __//  __   //  _\   //  __   / --__---_ _ > - -->

____/   //  /\  \ /  / /  //  /  \  \  / /  / _>_ __---__  -- >

\______/ \_/  \__\\_/  \_/ \_/   /__/_/  \_/         >-_ -- -_

/\____   /\____   /\____   /\_____

-- <_ __- -  /  __  \ /  __  \ /  __  \ /  ____/

--<  ___----_< _-- /  /__) //  /__) //  /__) //  /___

_<---  --<__---_ ___  /  __   //  _\   //  __   //  ____/

- <___ __-<-__  < /  /__) //  /  \  \  / /  //  /____

<  --_ ---<-- \______/ \_/   /__/_/  \_/ \______/[G]

... THE FASTEST BOARD AROUND FOR AMIGA AND PC !!!

NoW aLSo SuPPoRTiNG:

\*\ #[37mMeGaDRiVe#[0m /*/ #[37mFaMiCoM#[0m \*\ #[37mPC eNGiNe#[0m /*/

------------------------------------------------------------------------------

UPLOAD DATE AND TIME: 16.08.1992 08:17:55

#