______________________________________    �

\___  \___  \       \___  \___  \     \  _

/  /--/  /  /  /_/  /  / -/     /  /__/|(_)

/_____/_____/__/\/__/_____/__/__/__/\__\|(_)

� \_____\_____\__\/\__\_____\__\__\__\/Dt ----

Presents

Specification for the

����������������������������

ADVANCED AMIGA CHIP SET (AA)

����������������������������

SUPPLIED & TYPED BY FIREFLASH/SPONGE

CO/X\B/-\T:18

� Release Date 24.08.93 �

����������������������������

TABLE OF CONTENTS

-----------------

1.  SUMMERY OF NEW features for AA

2.  EXPLANATION of NEW Features

3.  List of Registers orderd by address

4.  List of registers orderd alphabetically

5.  NEW LISA DISPLAY MODES

1. SUMMERY OF NEW FEATURES FOR AA

---------------------------------

32 bit wide data bus supports input of 32-bit wide bitplane data and allows

doubling of memory bandwidth. Additional doubling of bandwidth can be

achieved

by using FAST page mode Ram. The same bandwidth enhancements are availble

for sprites. Also the maximum number of bitplanes useable in all modes was

increased to eight (8).

The color Palette has been expanded to 256 colors deep and 25 bits wide (8

RED,8 GREEN,8 BLUE,1 GENLOCK). This permits display of 256

simultaneouscolors in all resolutions. A palette of 16,777,216 colors are

available in all resolutions.

28Mhz clock input allows for cleaner definition of HIRES and SHRES pixels

ALICE`S clock genarator is synchronized by means of LISA`s 14MHz and SCLK

outputs, Genlock XCLK and XCLKEN pins have been eliminated (external MUX is

now required).

A new register bit allows sprites to appear in the screen border regions

(BRDRSPRT).

A bitplane mask field of 8 bits allows an address offset into the color

palette.

Two 4-bit mask fields do the same for odd and even sprites.

In Dual Playfield modes,2 4-bitplane playfields are now possible in all

resolutions.

Two Extra high-order playfield scrollbits allow seamless scrolling of up to

64 bit wide bitplanes in all resolutions. Resolution of bitplane scroll,

display window,and horizontal sprite position has been improved to 35ns in

all resolutions.

A new 8bitplane HAM mode has been created, 6 for colors and 2 for control

bits. All HAM modes are available in all resolutions (not just LORES as

before).

A RST_input pin has been added, which resets all the bits contained in

reg-isters that were new for

ECSorLISA:BPLCON3,BPLCON4,CLXCON2,DIWHIGH,FMODE.

Sprite resolution can be set to LORES,HIRES,SHRES,independant of bitplane

resolution.

Attached Sprites are now available in all resolutions.

Hardware Scan Doubling support has been added for bitplanes and sprites.

This is intended to allow 15KHz screens to be intelligently displayed on a

31KHz monitor and share the display with 31KHz screens.

2. EXPLANATION OF NEW FEATURES

------------------------------

Bitplanes

---------

There are now 8 bitplanes instaed of 6. In single playfield modes they can

address 256 colors instead of 64. As long as the memory architecture can

support the bandwidth, all 8 bitplanes are available in all 3 resolutions

In the same vein, 4+4 bitplane dualplayfield is available in all 3

resolutions, unless

bitplane scan-doubling is enabled, In which case both playfields share the

same bitplane modulus register. Bits 15 thru 8 of  BPLCON14 comprise an 8

bit mask for the 8 bitplane address, XOR`ing the individual bits.

This allows the copper to exchange color maps with a single instruction.

BPLCON1

now contains an 8 bit scroll value for each of the playfields. Granularity

of scroll now extends down to 35nSec.(1 SHRES pixel), and scroll can delay

playfield thru 32 bus cycles. Bits BPAGEM and BPL32 in new register FMODE

control size of bitplane data in BPL1DAT thru BPL8DAT.

The old 6 bitplane HAM mode, unlike before, works in HIRES and SHRES

resolutions.

As before bitplanes 5 and 6 control it`s function as follows:

|BP6|BP5|   RED  |  GREEN | BLUE             |

|--------------------------------------------|

| 0 | 0 | select new base register (1 of 16) |

|--------------------------------------------|

| 0 | 1 |  hold  |  hold  | modify           |

|--------------------------------------------|

| 1 | 0 | modify |  hold  |  hold            |

|--------------------------------------------|

| 1 | 1 |  hold  | modify |  hold            |

----------------------------------------------

There is a new 8 bitplane HAM (Hold and Modify) mode. This mode is invoked

when BPU field in BPLCON0 is set to 8 , and HAMEN is set. Bitplanes 1 and 2

are used as control bits analagous to the function of bitplanes 5 and 6 in

6 bitplane HAM mode:

|BP2|BP1|   RED  |  GREEN | BLUE             |

|--------------------------------------------|

| 0 | 1 | select new base register (1 of 64) |

|--------------------------------------------|

| 0 | 1 |  hold  |  hold  | modify           |

|--------------------------------------------|

| 1 | 0 | modify |  hold  |  hold            |

|--------------------------------------------|

| 1 | 1 |  hold  | modify |  hold            |

----------------------------------------------

Since only 6 bitplanes are available for modify data, the data is placed in

6 MSB. The 2 LSB are left unmodified, which allows creation of all

16,777,216 colors simultaneously, assuming one had a large enough screen

and picked one`s base registers judiciously. This HAM mode also works in

HIRES and SHRES modes.

For compatibility reasons EHB mode remains intact. Its existence is rather

moot in that we have more than enough colors in the color table to replace

its functionality. As before, EHB is invoked whenever SHRES = HIRES =

HAMEN= DPF = 0 and BPU = 6. Please note that starting with ECS DENISE

there is a bit in BPLCON2 which disables this mode (KILLEHB).

Bits PF2OF2,1,0 in BPLCON3 determine second playfield`s offset into the

color table. This is now necessary since playfields in DPF mode can have up

to 4 bitplanes. Offset value are as defined in register map.

BSCAN2 bit in FMODE enables bitplane scan-doubling. When V0 bit of DIWSTRT

matches V0 of vertical beam counter, BPL1MOD contains the modulus for the

display line, else BPL2MODis used. When scan-doubled both odd and even

bitplanes use the same modulus on a given line, whereas in normal mode odd

bitplanes used BPL1MOD and even bitplanes used BPL2MOD. As a result Dual

Playfields screens will probably not display correctly when scan-doubled.

Sprites:

--------

Bits SPAGEM and SPR32 in FMODE whether size of sprite load datain

SPR0DATA(B) thru SPR7DATA(B) is 16,32, or 64 bits, analagous to bitplanes.

BPLCON3 contains several bits relating to sprite behavior. SPRES1 and

SPRES0 control sprite resolution, whether they conform to theECS standard

or override tp LORES,HIRES,or SHRES. BRDRSPRT, when high,allows sprites to

be visible in border areas. ESPRM7 thru ESPRM4 allow relocation of the even

sprite color map. OSPRM7 thru OSPRN4 allow relocation of the odd sprite

color map. In the case of attached sprites OSPRM bits are used.

SSCAN2 bit in FMODE enables sprite scan-doubling. When enabled, individual

SH10 bits in SPRxPOS registers control whether or not a given sprite is to

be scan-doubled. When V0 bit of SPRxPOS register matches V0 bit of vertical

beam counter, the given sprite`s DMA is allowed to proceed as before. If

the don`t match, then sprite DMA is disabled and LISA reuses the sprite

data

from the previous line. When sprites are scan-doubled, only the position

and control registers need be modified by the programmer; the data

registers need no modification.

NOTE: Sprite vertical start and stop positions must be of the same parity,

i.e. both odd or even.

Compatibility:

--------------

RST_pin resets all bits in all registers new to AA. These registers

include:

BPLCON3,BPLCON4,CLXCON2,DIWHIGH,FMODE.

ECSENA bit (formerly ENBPLCN3) is used to disable those register bits in

BPLCON3 that are never accessed by old copper lists, and in addition are

required by old style copper lists to be in their default

settings.Specifically ECSENA forces the following bits to their default low

settings: BRDRBLNK,BRDNTRAN,ZDCLKEN,EXTBLKEN, and BRDRSPRT.

CLXCON2 is reset by a write to CLXCON, so that old game programs will be

able to correctly detect collisions.

DIWHIGH is reset by writes to DIWSTRT or DIWSTOP. This is interlock is

inhertied from ECS Denise.

Genlock:

--------

Lots of new genlock features were added to ECS DENISE and arecarried over

to LISA. ZDBPEN in BPLCON2 allows any bitplane, selected by ZDBPSEL2,1,0,to

be used as a transparency mask (ZD pin mirrors contents of selected

bitplane). ZDCTEN disables the old COLOR00 is transparent mode, and allows

the bit31 position of each color in the color table to control

transparency.ZDCLKEN generates a 14MHz clock synchronized with the video

data that can be used by video post-processors. Finally, BRDNTRAN in

BPLCON3 generates an opaque border region which can be used to frame live

video.

Color Lookup Table:

-------------------

The color table has grown from 32 13-bit registers to 256 25-bit registers.

Several new register bits have been added to BPLCON3 to facilitate loading

the table with only 32 register addresses. LOCT, selects either the 16 MSB

or LSB for loading. Loading the MSB always loads the LSB as well for

compatibility, so when 24 bit colors are desired load LSB after MSB.

BANK2,1,0 of 8 32 address banks for loading as follows:

BANK2 | BANK1 | BANK0 | COLOR ADDRESS RANGE

--------------------------------------------

0   |   0   |   0   |  COLOR00 - COLOR1F

0   |   0   |   1   |  COLOR20 - COLOR3F

0   |   1   |   0   |  COLOR40 - COLOR5F

0   |   1   |   1   |  COLOR60 - COLOR7F

1   |   0   |   0   |  COLOR80 - COLOR9F

1   |   0   |   1   |  COLORA0 - COLORBF

1   |   1   |   0   |  COLORC0 - COLORDF

1   |   1   |   1   |  COLORE0 - COLORFF

RDRAM bit in BPLCON2 causes LISA to interpret all color table accesses as

reads.

Note: There is no longer any need to "scramble" SHRES color table entries.

This artifice is no longer required and pepole who bypass ECS graphics

library calls to do their own 28MHz graphics are to be pointed at and

publicly humiliated.

Collision

---------

A new register CLXCON2 contains 4 new bits. ENBP7 and ENBP6 are the enable

bits for bitplanes 7 and 8, respectively. Similarly, MVBP7 and MPBP8 are

their match value bits. CLXDAT is unchanged.

Horizontal Comparators

----------------------

All programmable comparators with the exception of VHPOSW have 35nSec

resolution.: DIWHIGH,HBSTOP,SPRCTL,BPLCON1. BPLCON1 has additional

high-order bits as well. Note that horizontal bit position representing

140nSec resolution has been changed to 3rd least significant bit,where

before it used to be a field`s LSB, For example, bit 00 in BPLCON1 used to

be named PF1H0 and now it`s called PF1H2.

Coercion of 15KHz to 31KHz:

---------------------------

We have added new hardware features to LISA to aid in properly displaying

15KHz and 31KHz viewports together on the same 31KHz display. LISA can

globally set sprite resolution to LORES,HIRES, or SHRES.

LISA will ignore SH10 compare bits in SPRxPOS when scan-doubling, thereby

allowing ALICE to use these bits individually set scan-doubling.

3. LIST OF REGISTERS ORDERED BY ADDRESS

---------------------------------------

Symbols Used:

& = Register used by DMA channel only.

% = Register used by DMA channel usually, processors sometimes.

+ = Address register pair. Low word uses DB1-DB15, High word DB0-DB4.

~ = Address not writable by the coprocessor unless COPCON bit 1 is set true

h = new for HiRes chip set.

p = new for IAA chip set.

A = Agnus/Alice chip set.

D = Denise/Lisa chip set.

P = Paula chip.

W = Write.

R = Read.

ER= Early read. This is a DMA transfer to RAM, from either the disk or from

the blitter. Ram timing requires data to be on the bus earlier than

microprocessor read cycles. These transfers are therefore initiated by

Agnus timing, rather than a read address on the register address bus

(RGA).

S = Strobe (Write address with no register bits).

PTL,PTH = 20 bit pointer that addresses DMA data. Must be reloaded by a

processor before use (Vertical blank for bit plane and sprite pointers.

and prior to starting the blitter for blitter pointers). (old chips -

18 bits).

LCL,LCH = 20 bit location (starting address) of DMAdata. Used to

automatically restart pointers. such as the Coprocessor program counter

(during vertical blank), and the audio sample counter (whenever the

audio lentgh count is finished), (Old chips - 18 bits).

MOD = 15 bit Modulo. A number that is automatically added to the memory

address at the end of each line to generate the address for the

beginning of the next line. This allows the blitter (or the display

window) to operate on (or display) a window of data that is smaller

than the actual picture in memory. (memory map) Uses 15 bits, plus

sign extended.

NAME        ADDR   R/W CHIP(s)   FUNCTION

---------------------------------------------------------------------------

BPTDDAT   & ~000   ER  A         Blitter dest. early read (dummy address)

DMACONR     ~002   R   A     P   Dma control (and blitter status) read

VPOSR       ~004   R   A         Read vert most sig. bits (and frame flop

VHPOSR      ~006   R   A         Read vert and horiz position of beam

DSKDATR   & ~008   ER        P   Disk data early read (dummy address)

JOY0DAT     ~00A   R      D      Joystick-mouse 0 data (vert,horiz)

JOT1DAT     ~00C   R      D      Joystick-mouse 1 data (vert,horiz)

CLXDAT      ~00E   R      D      Collision data reg. (read and clear)

ADKCONR     ~010   R         P   Audio,disk control register read

POT0DAT     ~012   R         P   Pot counter pair 0 data (vert,horiz)

POT1DAT     ~014   R         P   Pot counter pair 1 data (vert,horiz)

POTINP      ~016   R         P   Pot pin data read

SERDATR     ~018   R         P   Serial port data and status read

DSKBYTR     ~01A   R         P   Disk data byte and status read

INTENAR     ~01C   R         P   Interrupt enable bits read

INTREQR     ~01E   R         P   Interrupt request bits read

DSKPTH    + ~020   W   A         Disk pointer (high 5 bits)

DSKPTL    + ~022   W   A         Disk pointer (low 15 bits)

DSKLEN      ~024   W         P   Disk lentgh

DSKDAT    & ~026   W         P   Disk DMA data write

REFPTR    & ~028   W   A         Refresh pointer

VPOSW       ~02A   W   A         Write vert most sig. bits(and frame flop)

VHPOSW      ~02C   W   A  D      Write vert and horiz pos of beam

COPCON      ~-2E   W   A         Coprocessor control reg (CDANG)

SERDAT      ~030   W         P   Serial port data and stop bits write

SERPER      ~032   W         P   Serial port period and control

POTGO       ~034   W         P   Pot count start,pot pin drive enable data

JOYTEST     ~036   W      D      Write to all 4 joystick-mouse counters at

once

STREQU    & ~038   S      D      Strobe for horiz sync with VB and EQU

STRVBL    & ~03A   S      D      Strobe for horiz sync with VB (vert blank)

STRHOR    & ~03C   S      D  P   Strobe for horiz sync

STRLONG   & ~03E   S      D      Strobe for identification of long horiz

line

BPLCON0     ~040   W  A          Blitter control reg 0

BPLCON1     ~042   W  A          Blitter control reg 1

BLTAFWM     ~044   W  A          Blitter first word mask for source A

BLTALWM     ~046   W  A          Blitter last word mask for source A

BLTCPTH   + ~048   W  A          Blitter pointer to source C (high 5 bits)

BLTCPTL   + ~04A   W  A          Blitter pointer to source C (low 15 bits)

BLTBPTH   + ~04C   W  A          Blitter pointer to source B (high 5 bits)

BLTBPTL   + ~04E   W  A          Blitter pointer to source B (low 15 bits)

BLTAPTH   + ~050   W  A          Blitter pointer to source A (high 5 bits)

BLTAPTL   + ~052   W  A          Blitter pointer to source A (low 15 bits)

BPTDPTH   + ~054   W  A          Blitter pointer to destn  D (high 5 bits)

BLTDPTL   + ~056   W  A          Blitter pointer to destn  D (low 15 bits)

BLTSIZE     ~058   W  A          Blitter start and size (win/width,height)

BPTCONOL  h ~05A   W  A          Blitter control 0 lower 8 bits (minterms)

BLTSIZV   h ~05C   W  A          Blitter V size (for 15 bit vert size)

BLTSIZH   h ~05E   W  A          Blitter H size & start (for 11 bit H size)

BLTCMOD     ~060   W  A          Blitter modulo for source C

BLTBMOD     ~062   W  A          Blitter modulo for source B

BLTAMOD     ~064   W  A          Blitter modulo for source A

BLTDMOD     ~066   W  A          Blitter modulo for destn  D

~068

~06a

~06c

~06e

BLTCDAT   & ~070   W  A          Blitter source C data reg

BLTBDAT   & ~072   W  A          Blitter source B data reg

BLTADAT   & ~074   W  A          Blitter source A data reg

~076

SPRHDAT   &h 078   W  A          Ext logic UHRES sprite pointer and data

identifier

(BPLHDAT)   ~07A   ????          ?????

LISAID    h ~07C   R     D       Chip revision level for Denise/Lisa

DSKSYNC     ~07E   W         P   Disk sync pattern reg for disk read

COP1LCH   +  080   W  A          Coprocessor first location reg

(high 5 bits)

COP1LCL   +  082   W  A          Coprocessor first location reg

(low 15 bits)

COP2LCH   +  084   W  A          Coprocessor second reg

(high 5 bits)

COP2LCL   +  086   W  A          Coprocessor second reg

(low 15 bits)

COPJMP1      088   S  A          Coprocessor restart at first location

COPJMP2      08A   S  A          Coprocessor restart at second location

COPINS       08C   W  A          Coprocessor inst fetch identify

DIWSTRT      08E   W  A  D       Display window start

(upper left vert-hor pos)

DIWSTOP      090   W  A  D       Display window stop

(lower right vert-hor pos)

DDFSTRT      092   W  A          Display bit plane data fetch start.hor pos

DDFSTOP      094   W  A          Display bit plane data fetch stop.hor pos

DMACON       096   W  A      P   DMA control write (clear or set)

CLXCON       098   W     D       Collision control

INTENA       09A   W         P   Interrupt enable bits (clear or set bits)

INTREQ       09C   W         P   Interrupt request bits (clear or set bits)

ADKCON       09E   W         P   Audio,disk,UART,control

AUD0LCH   +  0A0   W  A          Audio channel 0 location (high 5 bits)

AUD0LCL   +  0A2   W  A          Audio channel 0 location (low 15 bits)

AUD0LEN      0A4   W         P   Audio channel 0 lentgh

AUD0PER      0A6   W         P   Audio channel 0 period

AUD0VOL      0A8   W         P   Audio channel 0 volume

AUD0DAT   &  0AA   W         P   Audio channel 0 data

0AC

0AE

AUD1LCH   +  0B0   W  A          Audio channel 1 location (high 5 bits)

AUD1LCL   +  0B2   W  A          Audio channel 1 location (low 15 bits)

AUD1LEN      0B4   W         P   Audio channel 1 lentgh

AUD1PER      0B6   W         P   Audio channel 1 period

AUD1VOL      0B8   W         P   Audio channel 1 volume

AUD1DAT   &  0BA   W         P   Audio channel 1 data

0BC

0BE

AUD2LCH   +  0C0   W  A          Audio channel 2 location (high 5 bits)

AUD2LCL   +  0C2   W  A          Audio channel 2 location (low 15 bits)

AUD2LEN      0C4   W         P   Audio channel 2 lentgh

AUD2PER      0C6   W         P   Audio channel 2 period

AUD2VOL      0C8   W         P   Audio channel 2 volume

AUD2DAT   &  0CA   W         P   Audio channel 2 data

0CC

0CE

AUD3LCH   +  0D0   W  A          Audio channel 3 location (high 5 bits)

AUD3LCL   +  0D2   W  A          Audio channel 3 location (low 15 bits)

AUD3LEN      0D4   W         P   Audio channel 3 lentgh

AUD3PER      0D6   W         P   Audio channel 3 period

AUD3VOL      0D8   W         P   Audio channel 3 volume

AUD3DAT   &  0DA   W         P   Audio channel 3 data

0DC

0DE

BPL1PTH   +  0E0   W  A          Bit plane pointer 1 (high 5 bits)

BPL1PTL   +  0E2   W  A          Bit plane pointer 1 (low 15 bits)

BPL2PTH   +  0E4   W  A          Bit plane pointer 2 (high 5 bits)

BPL2PTL   +  0E6   W  A          Bit plane pointer 2 (low 15 bits)

BPL3PTH   +  0E8   W  A          Bit plane pointer 3 (high 5 bits)

BPL3PTL   +  0EA   W  A          Bit plane pointer 3 (low 15 bits)

BPL4PTH   +  0EC   W  A          Bit plane pointer 4 (high 5 bits)

BPL4PTL   +  0EE   W  A          Bit plane pointer 4 (low 15 bits)

BPL5PTH   +  0F0   W  A          Bit plane pointer 5 (high 5 bits)

BPL5PTL   +  0F2   W  A          Bit plane pointer 5 (low 15 bits)

BPL6PTH   +  0F4   W  A          Bit plane pointer 6 (high 5 bits)

BPL6PTL   +  0F6   W  A          Bit plane pointer 6 (low 15 bits)

BPL7PTH   +  0F8   W  A          Bit plane pointer 7 (high 5 bits)

BPL7PTL   +  0FA   W  A          Bit plane pointer 7 (low 15 bits)

BPL8PTH   +  0FC   W  A          Bit plane pointer 8 (high 5 bits)

BPL8PTL   +  0FE   W  A          Bit plane pointer 8 (low 15 bits)

BPLCON0      100   W  A  D       Bit plane control reg (misc control bits)

BPLCON1      102   W     D       Bit plane control reg (scroll val PF1,PF2)

BPLCON2      104   W     D       Bit plane control reg (priority control)

BPLCON3      106   W     D       Bit plane control reg (enhanced features)

BPL1MOD      108   W  A          Bit plane modulo (odd planes,or active-

fetch lines if bitplane scan-doubling is

enabled

BPL2MOD      10A   W  A          Bit plane modulo (even planes or inactive-

fetch lines if bitplane scan-doubling is

enabled

BPLCON4   p  10C   W     D       Bit plane control reg (bitplane and sprite

masks)

CLXCON2   p  10e   W     D       Extended collision control reg

BPL1DAT   &  110   W     D       Bit plane 1 data (parallel to serial con-

vert)

BPL2DAT   &  112   W     D       Bit plane 2 data (parallel to serial con-

vert)

BPL3DAT   &  114   W     D       Bit plane 3 data (parallel to serial con-

vert)

BPL4DAT   &  116   W     D       Bit plane 4 data (parallel to serial con-

vert)

BPL5DAT   &  118   W     D       Bit plane 5 data (parallel to serial con-

vert)

BPL6DAT   &  11a   W     D       Bit plane 6 data (parallel to serial con-

vert)

BPL7DAT   &p 11c   W     D       Bit plane 7 data (parallel to serial con-

vert)

BPL8DAT   &p 11e   W     D       Bit plane 8 data (parallel to serial con-

vert)

SPR0PTH   +  120   W  A          Sprite 0 pointer (high 5 bits)

SPR0PTL   +  122   W  A          Sprite 0 pointer (low 15 bits)

SPR1PTH   +  124   W  A          Sprite 1 pointer (high 5 bits)

SPR1PTL   +  126   W  A          Sprite 1 pointer (low 15 bits)

SPR2PTH   +  128   W  A          Sprite 2 pointer (high 5 bits)

SPR2PTL   +  12A   W  A          Sprite 2 pointer (low 15 bits)

SPR3PTH   +  12C   W  A          Sprite 3 pointer (high 5 bits)

SPR3PTL   +  12E   W  A          Sprite 3 pointer (low 15 bits)

SPR4PTH   +  130   W  A          Sprite 4 pointer (high 5 bits)

SPR4PTL   +  132   W  A          Sprite 4 pointer (low 15 bits)

SPR5PTH   +  134   W  A          Sprite 5 pointer (high 5 bits)

SPR5PTL   +  136   W  A          Sprite 5 pointer (low 15 bits)

SPR6PTH   +  138   W  A          Sprite 6 pointer (high 5 bits)

SPR6PTL   +  13A   W  A          Sprite 6 pointer (low 15 bits)

SPR7PTH   +  13C   W  A          Sprite 7 pointer (high 5 bits)

SPR7PTL   +  13E   W  A          Sprite 7 pointer (low 15 bits)

SPR0POS   %  140   W  A  D       Sprite 0 vert-horiz start pos data

SPR0CTL   %  142   W  A  D       Sprite 0 position and control data

SPR0DATA  %  144   W     D       Sprite 0 image data register A

SPR0DATB  %  146   W     D       Sprite 0 image data register B

SPR1POS   %  148   W  A  D       Sprite 1 vert-horiz start pos data

SPR1CTL   %  14A   W  A  D       Sprite 1 position and control data

SPR1DATA  %  14C   W     D       Sprite 1 image data register A

SPR1DATB  %  14E   W     D       Sprite 1 image data register B

SPR2POS   %  150   W  A  D       Sprite 2 vert-horiz start pos data

SPR2CTL   %  152   W  A  D       Sprite 2 position and control data

SPR2DATA  %  154   W     D       Sprite 2 image data register A

SPR2DATB  %  156   W     D       Sprite 2 image data register B

SPR3POS   %  158   W  A  D       Sprite 3 vert-horiz start pos data

SPR3CTL   %  15A   W  A  D       Sprite 3 position and control data

SPR3DATA  %  15C   W     D       Sprite 3 image data register A

SPR3DATB  %  15E   W     D       Sprite 3 image data register B

SPR4POS   %  160   W  A  D       Sprite 4 vert-horiz start pos data

SPR4CTL   %  162   W  A  D       Sprite 4 position and control data

SPR4DATA  %  164   W     D       Sprite 4 image data register A

SPR4DATB  %  166   W     D       Sprite 4 image data register B

SPR5POS   %  168   W  A  D       Sprite 5 vert-horiz start pos data

SPR5CTL   %  16A   W  A  D       Sprite 5 position and control data

SPR5DATA  %  16C   W     D       Sprite 5 image data register A

SPR5DATB  %  16E   W     D       Sprite 5 image data register B

SPR6POS   %  170   W  A  D       Sprite 6 vert-horiz start pos data

SPR6CTL   %  172   W  A  D       Sprite 6 position and control data

SPR6DATA  %  174   W     D       Sprite 6 image data register A

SPR6DATB  %  176   W     D       Sprite 6 image data register B

SPR7POS   %  178   W  A  D       Sprite 7 vert-horiz start pos data

SPR7CTL   %  17A   W  A  D       Sprite 7 position and control data

SPR7DATA  %  17C   W     D       Sprite 7 image data register A

SPR7DATB  %  17E   W     D       Sprite 7 image data register B

COLOR00      180   W     D       Color table 00

COLOR01      182   W     D       Color table 01

COLOR02      184   W     D       Color table 02

COLOR03      186   W     D       Color table 03

COLOR04      188   W     D       Color table 04

COLOR05      18A   W     D       Color table 05

COLOR06      18C   W     D       Color table 06

COLOR07      18E   W     D       Color table 07

COLOR08      190   W     D       Color table 08

COLOR09      192   W     D       Color table 09

COLOR10      194   W     D       Color table 10

COLOR11      196   W     D       Color table 11

COLOR12      198   W     D       Color table 12

COLOR13      19A   W     D       Color table 13

COLOR14      19C   W     D       Color table 14

COLOR15      19E   W     D       Color table 15

COLOR16      1A0   W     D       Color table 16

COLOR17      1A2   W     D       Color table 17

COLOR18      1A4   W     D       Color table 18

COLOR19      1A6   W     D       Color table 19

COLOR20      1A8   W     D       Color table 20

COLOR21      1AA   W     D       Color table 21

COLOR22      1AC   W     D       Color table 22

COLOR23      1AE   W     D       Color table 23

COLOR24      1B0   W     D       Color table 24

COLOR25      1B2   W     D       Color table 25

COLOR26      1B4   W     D       Color table 26

COLOR27      1B6   W     D       Color table 27

COLOR28      1B8   W     D       Color table 28

COLOR29      1BA   W     D       Color table 29

COLOR30      1BC   W     D       Color table 30

COLOR31      1BE   W     D       Color table 31

HTOTAL    h  1C0   W  A          Highest number count in horiz line

(VARBEAMEN = 1)

HSSTOP    h  1C2   W  A          Horiz line pos for HSYNC stop

HBSTRT    h  1C4   W  A  D       Horiz line pos for HBLANK start

HBSTOP    h  1C6   W  A  D       Horiz line pos for HBLANK stop

VTOTAL    h  1C8   W  A          Highest numbered vertical line

(VARBEAMEN = 1)

VSSTOP    h  1CA   W  A          Vert line for VBLANK start

VBSTRT    h  1CC   W  A          Vert line for VBLANK start

VBSTOP    h  1CE   W  A          Vert line for VBLANK stop

SPRHSTRT  h  1D0   W  A          UHRES sprite vertical start

SPRHSTOP  h  1D2   W  A          UHRES sprite vertical stop

BPLHSTRT  h  1D4   W  A          UHRES bit plane vertical stop

BPLHSTOP  h  1D6   W  A          UHRES bit plane vertical stop

HHPOSW    h  1D8   W  A          DUAL mode hires H beam counter write

HHPOSR    h  1DA   R  A          DUAL mode hires H beam counter read

BEAMCON0  h  1DC   W  A          Beam counter control register

(SHRES,UHRES,PAL)

HSSTRT    h  1DE   W  A          Horizontal sync start (VARHSY)

VSSTRT    h  1E0   W  A          Vertical sync start (VARVSY)

HCENTER   h  1E2   W  A          Horizontal pos for vsync on interlace

DIWHIGH   h  1E4   W  A  D       Display window upper bits for start/stop

BPLHMOD   h  1E6   W  A          UHRES bit plane modulo

SPRHPTH  +h  1E8   W  A          UHRES sprite pointer (high 5 bits)

SPRHPTL  +h  1EA   W  A          UHRES sprite pointer (low 15 bits)

BPLHPTH  +h  1EC   W  A          VRam (UHRES) bitplane pointer (hi 5 bits)

BPLHPTL  +h  1EE   W  A          VRam (UHRES) bitplane pointer (lo 15 bits)

RESERVED     1F0 - 1FA

FMODE     p  1FC   W  A  D       Fetch mode register

NO-OP(NULL)  1FE                 Can also indicate last 2 or 3 refresh

cycles or the restart of the COPPER after

lockup.

4. LIST OF REGISTERS ORDERED ALPHABETICALLY

-------------------------------------------

P  =  New register in Pandora chip set

p  =  Stuff added or changed in hires chips

H  =  New register in hires chips

h  =  stuff added or changed in hires chips

A  =  Agnus/Alice chip

D  =  Denise/Lisa chip

P  =  Paula chip

W  =  Write

R  =  Read

ER =  Early read. This is a DMA data transfer to RAM, from either the disk

or from the blitter, Ram timing requires data to be on the bus

earlier than microprocessor read cycles. These transfers are

therefore initiated by Agnus timing, rather than a read address on

the register address bus (RGA).

NAME   rev ADDR type chip Description

---------------------------------------------------------------------------

ADKCON     09E   W   P    Audio,Disk,Uart,Control write

ADKCONR    010   R   P    Audio,Disk,Uart,Control read

BITS  USE

----  ----------------------------------------------------

15    SET/CLEAR  Set/clear control bit.determines if bits

written with a 1 get set or cleared.bits

written with a zero are always unchanged.

14-13 PRECOMP 1-0   CODE   PRECOMP VALUE

----   -------------

00    none

01    140 ns

10    280 ns

11    560 ns

12    MFMPREC    (1 = MFM precomp / 0 = GCR precomp)

11    UARTBRK    Forces a UART break (clears TXD) if true

10    WORDSYNC   Enables disk read synchronizing on a word

equal to DISK SYNC CODE, Located in

address DSKSYNC (7E).

09    MSBSYNC    Enables disk read synchrinizing on the                                   MSB (most signif bit) appl type GCR

08    FAST       Disk data clock rate control 1=fast(2us)

0=slow(4us)

(Fast for MFM or 2us,slow for 4us GCR)

07    USE3PN     Use audio channel 3 to modulate nothing

06    USE2P3     Use audio channel 2 to modulate period of

channel 3

05    USE1P2     Use audio channel 1 to modulate period

of channel 2

04    USE0P1     Use audio channel 0 to modulate period

of channel 1

03    USE3VN     Use audio channel 3 to modulate nothing                  02    USE2V3     Use audio channel 2 to modulate volume

of channel 3

01    USE1V2     Use audio channel 1 to modulate volume

of channel 2

00    USE0V1     Use audio channel 0 to modulate volume

of channel 1

NOTE: If both period and volume aremodulated on the same

channel,the period and volume will be alternated.

first AUDxDAT word is used for V6-V0 of UDxVOL.

second AUDxDAT word is used for P15-P0 of AUDxPER.

This alternating sequence is repeated.

AUDxLCH h 0A0    W   A            Audio channel x location (high 5 bits)

(old-3 bits)

AUDxLCL   0A2    W   A            Audio channel x location (low 15 bits)

This pair of registers contains the 20 bit starting

address(location) of audio channel x (x=0,1,2,3)DMA

data. This is not a pointer reg and therfore only

needs to be reloaded if a diffrent memory location

is to be outputted.

AUDxLEN   0A4    W   P            Audio channel x lentgh

This reg contains the lentgh (number of words) of

audio channel x DMA data.

AUDxPER h 0A6    W   P            Audio channel x period

This reg contains the period (rate) of audio channel

x DMA data transfer.

The minimum period is 124 clocks. This means that

the smallest number that should be placed in this

reg is 124.

AUDxVOL   0A8    W   P            Audio channel x volume

This reg contains the volume setting for audio

channel x. Bits 6,5,4,3,2,1,0 specify 65 linear

volume levels as shown below.

BITS    USE

----    --------------------------------------------------

-15-07  Not used

06      Forces volume to max (64 ones,no zeros)

05-00   Sets one of the 64 levels (000000 = no output

(111111 = 63 ones, one zero)

AUDxDAT   0AA    W   P            Audio channel x data

This reg is the audio channel x (x=0,1,2,3) DMA

data buffer. It contains 2 bytes of data (each

byte is a twos complement signed integer) that

are outputed sequentially (with digital to analog

conversion)to the audio output pins. With maximum

volume, each byte can drive the audio outputs

with 0.8 volts(peak to peak,typ). The audio DMA

channel controller automatically transfers data

to this reg from RAM. The processor can also

write directly to this reg. When the DMA data is

finished (words outputted=lentgh)and the data in

this reg has been used, an audio channel interrupt

request is set.

BEAMCON0 h 1DC  W    A            Beam counter control bits

BIT     FUNCTION

---     --------------------------------------------------

15      (unused)

14      HARDDIS

13      LPENDIS

12      VARVBEN

11      LOLDIS

10      CSCBEN

9      VARVSYEN

8      VARHSYEN

7      VARBEAMEN

6      DUAL

5      PAL

4      VARCSYEN

3      (unused, formerly BLANKEN)

2      CSYTRUE

1      VSYTRUE

0      HSYTRUE

HARDDIS = This bit is used to disable the hardwire vertical horizontal

window limits. It is cleared upon reset.

LPENDIS = When this bit is a low and LPE (BPLCON0,BIT 3) is enabled, the

light-pen latched value(beam hit position) will be read by

VHPOSR,VPOSR and HHPOSR. When the bit is a high the light-pen

latched value is ignored and the actual beam counter position is           read by  VHPOSR,VPOSR, and HHPOSR.

VARVBEN = Use the comparator generated vertical blank (from VBSTRT,VBSTOP)           to run the internal chip stuff-sending RGA signals to Denise,           starting sprites,resetting light pen. It also disables the hard

stop on the vertical display window.

LOLDIS  = Disable long line/short toggle. This is useful for DUAL mode

where even multiples are wanted, or in any single display

where this toggling is not desired.

CSCBEN  = The variable composite sync comes out on the HSY pin, and the

variable conosite blank comes out on the VSY pin. The idea is

to allow all the information to come out of the chip for a

DUAL mode display. The normal monitor uses the normal composite

sync, and the variable composite sync &blank come out the HSY &

VSY pins. The bits VARVSTEN & VARHSYEN (below) have priority over

this control bit.

VARVSYEN= Comparator VSY -> VSY pin. The variable VSY is set vertically on

VSSTRT, reset vertically on VSSTOP, with the horizontal position

for set set & reset HSSTRT on short fields (all fields are short

if LACE = 0) and HCENTER on long fields (every other field if

LACE = 1).

VARHSYEN= Comparator HSY -> HSY pin. Set on HSSTRT value, reset on HSSTOP

value.

VARBEAMEN=Enables the variable beam counter comparators to operate

(allowing diffrent beam counter total values) on the main horiz

counter. It also disables hard display stops on both horizontal

and vertical.

DUAL    = Run the horizontal comparators with the alternate horizontal beam

counter, and starts the UHRES pointer chain with the reset of

this counter rather than the normal one. This allows the UHRES

pointers to come out more than once in a horizontal line,

assuming there is some memory bandwidth left (it doesn`t work in

640*400*4 interlace mode) also, to keep the two displays synced,

the horizontal line lentghs should be multiples of each other.

If you are amazingly clever, you might not need to do this.

PAL     = Set appropriate decodes (in normal mode) for PAL. In variable

beam counter mode this bit disables the long line/short line

toggle- ends up short line.

VARCSYEN= Enables CSY from the variable decoders to come out the CSY

(VARCSY is set on HSSTRT match always, and also on HCENTER

match when in vertical sync. It is reset on HSSTOP match when VSY

and on both HBSTRT &HBSTOP matches during VSY. A reasonable

composite can be generated by setting HCENTER half a horiz line

from HSSTRT, and HBSTOP at (HSSTOP-HSSTRT) before HCENTER, with

HBSTRT at (HSSTOP-HSSTRT) before .... see below

---------------------------------------------------------------------------

PAGE 19 MISSING (SORRY!)

---------------------------------------------------------------------------

BLTCON0    040  W   A           Blitter control register 0

BLTCON0L H 05A  W   A           Blitter control register 0 (lower 8 bits)

This is to speed up software - the upper bits are

often the same.

BLTCON1  h 042  W   A           Blitter control register 1

These two control registers are used together to

control blitter operations. There are 2 basic

modes, area and line, which are selected by bit 0

of BLTCON1, as shown below.

AREA MODE ("normal")     LINE MODE (line draw)

--------------------------------------------------

BIT#  BLTCON0  BLTCON1    BIT#  BLTCON0  BLTCON1

--------------------------------------------------

15    ASH3     BSH3       15    ASH3     BSH3

14    ASH2     BSH2       14    ASH2     BSH2

13    ASH1     BSH1       13    ASH1     BSH1

12    ASA0     BSH0       12    ASH0     BSH0

11    USEA      0         11     1         0

10    USEB      0         10     0         0

09    USEC      0         09     1         0

08    USED      0         08     1         0

07    LF7      DOFF       07    LF7       DPFF

06    LF6       0         06    LF6       SIGN

05    LF5       0         05    LF5       OVF

04    LF4      EFE        04    LF4       SUD

03    LF3      IFE        03    LF3       SUL

02    LF2      FCI        02    LF2       AUL

01    LF1      DESC       01    LF1       SING

00    LF0      LINE(=0)   00    LF0       LINE(=1)

ASH3-0    Shift value of A source

BSH3-0    Shift value of B source and line texture

USEA      Mode control bit to use source A

USEB      Mode control bit to use source B

USEC      Mode control bit to use source C

USED      Mode control bit to use destination D

LF7-0     Logic function minterm select lines

EFE       Exclusive fill enable

IFE       Inclusive fill enable

FCI       Fill carry input

DESC      Descending (dec address)control bit

LINE      Line mode control bit

SIGN      Line draw sign flag

OVF       Line/draw r/l word overflow flag

SUD       Line draw, Sometimes up or down (=AUD)

SUL       Line draw, Sometimes up or left

AUL       Line draw, Always up or left

SING      line draw, Single bit per horiz line

DOFF      Disables the D output- for external ALUs

The cycle occurs normally, but the data

bus is tristate (hires chips only)

BLTSIZH  h 05E  W   A           Blitter H size & start (11 bit width)

BIT# 15 14 13 12 11 10  09 08 07 06 05 04 03 02 01 00

x  x  x  x  x w10 w9 w8 w7 w6 w5 w4 w3 w2 w1 w0

BLTSIZV  h 05C  W   A           Blitter V size (15 bit height)

BIT# 15 14  13  12  11  10  09 08 07 06 05 04 03 02 01 00

x h14 h13 h12 h11 h10 h9 h8 h7 h6 h5 h4 h3 h2 h1 h0

These are the blitter size regs for blits larger than

the earlier chips could accept. The original commands

are retained for compatibility. BLTSIZV should be

written first, followed by BLTSIZH, which starts the

blitter. BLTSIZV need not be rewritten for subsequent

bits if the vertical size is the same.

Max size of blit 32k pixels * 32k lines, x s should be

written to 0 for upward compatibility.

BPLHDAT  h 07A  W               Ext logic UHRES bit plane identifier

BPLHMOD  h 1E6  W   A           Uhres bit plane modulo

This is the number (sign extended) that is added to the

UHRES bit plane pointer (BPLHPTL,H) every line, and

then another 2 is added, just like the other modulos.

BPLHPTH  h 1EC  W   A           UHRES (VRAM) bit plane pntr (high 5 bits)

BPLHPTL  h 1EE  W   A           UHRES (VRAM) bit plane pntr (low 15 bits)

When UHRES is enabled, this pointer comes out on the

2nd 'free' cycle after the start of each horizontal

line. It`s modulo is added every time it comes out.

'free' means priority above the copper and below the

fixed stuff (audio,sprites....).

BPLHDAT comes out as an identifier on the RGA lines when

the pointer address is valid so that external detectors

can use this to do the special cycle for the VRAMs, The

SHRHDAT gets the first and third free cycles.

BPLHSTOP p 1D6  W   A           UHRES bit plane vertical stop

BIT#   name

----   ----

15     BPLHWRM

14-11  Unused

10-0   V10-V0

BPLHWRM = Swaps the polarity of ARW* when the BPLHDAT

comes out so that external devices can detect the RGA

and put things into memory (ECS and later versions).

BPLHSTRT h 1D4  W   A           UHRES bit plane vertical stop

This controls the line when the data fetch starts for

the BPLHPTH,L pointers. V10-V0 on DB10-0.

BPLxPTH   0E0   W   A           Bit plane x pointer (high 5 bits)

0E8            x=1,2,3,4,5,6,7,8

0EC

0F0

0F4

p 0F8

p 0FC

BPLxPTL   0E2  W    A           Bit plane pointer (low 15 bits)

0EA          Address of bit plane x  (x=1,2,3,4,5,6,7,8) DMA data.

0EE          This pointer must be reinitialized by the processor or

0F2          coprocessor to point to the beginning of bit plane data

0F6          every vertical blank time.

p 0FA

p 0FE

BPLxDAT   110  W    A           Bit plane x data (parallel to serial convert)

112          These regs recieve the DMA data fetched from RAM by the

114          bit plane address pointers described above.

116          They may also be rewritten by either micro.

118          they act as a 8 word parallel to serial buffer for up

11A          to 8 memory 'bit planes'. x=1-8 the parallel to serial

p 11C          conversion id triggered whenever bit plane #1 is

p 11E          written, indicing the completion of all bit planes for

that word (16/32/64 pixels). The MSB is output first,

and is therefore always on the left.

BPL1MOD   108  W    A           Bit plane modulo (odd planes)

BPL2MOD   10A  W    A           BIt plane modulo (even planes)

These registers contain the modulos for the odd and

even bit planes. A modulo is a number that is automa-

itcally added to the address at the end of each line,

in order that the address then points to the start of

the next line. Since they have seperate modulos, the

odd and even bit planes may have sizes that are

different from each other, as well as different from

the display window size.

If scan-doubling is enabled, BPL1MOD serves as the

primary bitplane modulos and BPL2MOD serves as the

alternate. Lines whose LSBs of beam counter and DIWSTRT

match are designated primary, whereas lines whose LSBs

don`t match are designated alternate.

BPLCON0 p 100  W    A  D        Bit plane control reg. (misc, control bits)

BPLCON1 p 102  W       D        Bit plane control reg. (horiz, scroll counter)

BPLCON2 p 104  W       D        Bit plane control reg. (new control bits)

BPLCON3 p 106  W       D        Bit plane control reg. (enhanced features)

BPLCON4 p 10c  W       D        Bit plane control reg. (display masks)

These regs control the operation of the bit planes and

various aspects of the display.

BIT#   BPLCON0   BPLCON1   BPLCON2   BPLCON3    BPLCON4

----   -------   -------   -------   -------    -------

15     HIRES     PF2H7=0   X         BANK2=0    BPLAM7=0

14     BPU2      PF2H6=0   ZDBPSEL2  BANK1=0    BPLAM6=0

13     BPU1      PF2H1=0   ZDBPSEL1  BANK0=0    BPLAM5=0

12     BPU0      PF2H0=0   ZDBPSEL0  PF2OF2=0   BPLAM4=0

11     HAM       PF1H7=0   ZDBPEN    PF2OF1=1   BPLAM3=0

10     DPF       PF1H6=0   ZDCTEN    PF2OF0=1   BPLAM2=0

09     COLOR     PF1H1=0   KILLEHB   LOCT=0     BPLAM1=0

08     GAUD      PF1H0=0   RDRAM=0   X          BPLAM0=0

07     UHRES     PF2H5     SOGEN=0   SPRES1=0   ESPRM7=0

06     SHRES     PF2H4     PF2PRI    SPRES0=0   ESPRM6=0

05     BYPASS=0  PF2H3     PF2P2     BRDRBLNK=0 ESPRM5=0

04     BPU3=0    PF2H2     PF2P1     BRDNTRAN=0 ESPRM4=1

03     LPEN      PF1H5     PF2P0     X          OSPRM7=0

02     LACE      PF1H4     PF1P2     ZDCLKEN=0  OSPRM6=0

01     ERSY      PF1H3     PF1P1     BRDSPRT=0  OSPRM5=0

00     ECSENA=0  PF1H2     PF1P0     EXTBLKEN=0 OSPRM4=1

x = don`t care; but drive to 0 for upward compatibility!

HIRES = High resoloution (640*200/640*400 interlace) mode

BPUx  = Bit plane use code 0000-10000 (NONE thru 8 inclusive)

HAM   = Hold and modify mode, now using either 6 or 8 bit planes.

New HAM mode is invoked when this bit is set and BPU = 1000.

DPF   = Double playfield (PFI=odd FP2= even bit planes)

now available in all resoloutions.

(If BPU=6 and HAM=0 and DPF=0 a special mode is defined that allows

bitplane 6 to cause an intensity reduction of the other 5 bitplanes

The color register output selected by 5 bitplanes is shifted to half

intensity  by the 6th bit plane. This is called EXTRA-HALFBRITE

Mode.

COLOR = Enables color burst output signal

GAUD  = Genlock audio enable. This level appears on the ZD pin

on denise during all blanking periods, unless ZDCLK bit is set

UHRES = Ultrahi res enables the UHRES pointers (for 1k*1k) (also

needs bits in DMACON (hires chips only).

Disables hard stops for vert, horiz display windows

SHRES = Super hi-res mode (35ns pixel width)

BYPASS = Bitplanes are scrolled and prioritized normally, but

bypass color table and 8 bit wide data appear on R(7:0).

LPEN = Light pen enable (reset on power up)

LACE = Interlace enable (reset om power up)

ERSY = External resync (HSYNC, VSYNC pads become inputs)

(reset on power up)

ECSENA = When low (default), the following bits in BPLCON3 are

disabled: BRDRBLNK,BRDNTRAN,ZDCLKEN,BRDSPRT, and EXTBLKEN.

These 5 bits can always be set by writing to BPLCON3, however

there effects are inhibited until ECSENA goes high. This allows

rapid context switching between pre-ECS viewports and new ones.

PF2Hx = Playfield 2 horizontal scroll code, x=0-7

PF1Hx = Playfield 1 horizontal scroll code, x=0-7

where PFyH0=LSB=35ns SHRES pixel (bits have been renamed, old

PFyH0 now PFyH2, ect). Now that the scroll range has been

quadrupled to allow for wider (32 or 64 bits) bitplanes.

ZDBPSELx =3 bit field which selects which bitplane is to be used

for ZD when ZDBBPEN is set;000 selects BB1 and 111 selects BP8.

ZDBPEN = Causes ZD pin to mirror bitplane selected by ZDBPSELx

bits. This does not disable the ZD mode defined by ZDCTEN, but

rather is "ored" with it.

ZDCTEN = Causes ZD pin to mirror bit #15 of the active entry in

high color table. When ZDCTEN is reset ZD reverts to mirroring

color (0).

KILLEHB = Disables extra half brite mode.

RDRAM = Causes color table address to read the color table instead

of writing to it.

SOGEN = When set causes SOG output pin to go high

PF2PRI = Gives playfield 2 priority over playfield 1.

PF2Px = Playfield 2 priority code (with resp. to sprites).

PF1Px = Playfield 1 priority code (with resp. to sprites).

BANKx = Selects one of eight color banks, x=0-2.

PF20Fx = Determine bit plane color table offset whe playfield 2

has priority in dual playfield mode:

PF20F || AFFECTED BITPLANE ||OFFSET

-------------------------------------------------------

| 2 | 1 | 0 || 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 ||(decimal

-------------------------------------------------------

| 0 | 0 | 0 || - | - | - | - | - | - | - | - || none

| 0 | 0 | 1 || - | - | - | - | - | - | 1 | - || 2

| 0 | 1 | 0 || - | - | - | - | - | 1 | - | - || 4

| 0 | 1 | 1 || - | - | - | - | - | 1 | - | - || 8 (default)

| 1 | 0 | 0 || - | - | - | 1 | - | - | - | - || 16

| 1 | 0 | 1 || - | - | 1 | - | - | - | - | - || 32

| 1 | 1 | 0 || - | 1 | - | - | - | - | - | - || 64

| 1 | 1 | 1 || 1 | - | - | - | - | - | - | - || 128

LOCT = Dictates that subsequent color palette values will be

written to a second 12- bit color palette, constituting the

RGB low minus order bits. Writes to the normal hi monus order

color palette automattically copied to the low order for back-

wards compatibility.

SPRESx = Determine resolution of all 8 sprites (x=0,1);

-------------------------------------------------------------

SPRES1 | SPRES0 | SPRITE RESOLUTION

-------------------------------------------------------------

0      | 0      | ECS defaults (LORES,HIRES=140ns,SHRES=70ns)

0      | 1      | LORES (140ns)

1      | 0      | HIRES (70ns)

1      | 1      | SHRES (35ns)

BRDRBLNK = "Border area" is blanked instead of color (0).

Disabled when ECSENA low.

BRDNTRAN = "Border area" is non minus transparant (ZD pin is low

when border is displayed).

Disabled when ECSENA low.

ZDCLKEN = ZD pin outputs a 14MHz clock whose falling edge coincides

with hires (7MHz) video data. this bit when set disables all other

ZD functions.

Disabled when ESCENA low.

BRDRSPRT = enables sprites outside the display window.

disabled when ESCENA low.

EXTBLKEN = causes BLANK output to be programmable instead of

reflecting internal fixed decodes.

disabled when ESCENA low.

BPLAMx = This 8 bit field is XOR`ed with the 8 bit plane color

address, thereby altering the color address sent to the color

table (x=1-8)

ESPRMx = 4 Bit field provides the 4 high order color table address

bits for even sprites: SPR0,SPR2,SPR4,SPR6. Default value is 0001

binary. (x=7-4)

OSPRMx = 4 Bit field provides the 4 high order color table address

bits for odd sprites: SPR1,SPR3,SPR5,SPR7. Default value is 0001

binary. (x=7-4)

CLXCON    098  W   A            Collision control

This register controls which bitplanes are included

(enabled) in collision detection, and thier required

state if included. It also controls the individual

inclusion of odd numbered sprites in the collision

detection, by logically ORing them with thier

correspondingeven numbered sprite. Writing to this

register resets thebits in CLXCON2.

BIT# FUNCTION DESCRIPTION

---- -------- -----------

15   ENSP7    Enable Sprite 7 (ORed with Sprite 6)

14   ENSP5    Enable Sprite 5 (ORed with Sprite 4)

13   ENSP3    Enable Sprite 3 (ORed with Sprite2)

12   ENSP1    Enable Sprite 1 (ORed with Sprite 0)

11   ENSP6    Enable bit plane 6 (match reqd. for collision

10   ENSP5    Enable bit plane 5 (match reqd. for collision                    09   ENSP4    Enable bit plane 4 (match reqd. for collision

08   ENSP3    Enable bit plane 3 (match reqd. for collision

07   ENSP2    Enable bit plane 2 (match reqd. for collision

06   ENSP1    Enable bit plane 1 (match reqd. for collision

05   ENSP6    Match value for bit plane 6 collision

04   ENSP5    Match value for bit plane 5 collision

03   ENSP4    Match value for bit plane 4 collision

02   ENSP3    Match value for bit plane 3 collision

01   ENSP2    Match value for bit plane 2 collision

00   ENSP1    Match value for bit plane 1 collision

CLXCON2 P  10C  W    D          Extended collision control

This reg controls when bit planes 7 and 8 are included

in collision detection, and there required state if

included. Contents of this register are reset by a

write to CLXCON.

BITS INITIALIZED BY RESET

BIT#  FUNCTION DESCRIPTION

----  -------- -----------

15-08          unused

07    ENBP8    Enable bit plane 8 (match reqd. for

collision)

06    ENBP7    Enable bit plane 7 (match reqd. for

collision)

05-02          unused

01    MVBP8    Match value for bit plane 8 collision

00    MVBP7    Match value for bit plane 7 collision

note: Disable bit planes cannot prevent collisions.

Therefore if all bitplanes are disabled, collision

will be continuous, regardless of the match values.

CLXDAT   00E  R     D           Collision data reg. (read and clear)

This address reads (and clears) the collision

detection reg. the bit assignments are below

NOTE: Playfield 1 is all odd numbered enabled bit planes.

Playfield 2 is all even numbred enabled bit planes.

BIT#   COLLISIONS REGISTERED

----   ---------------------

15     not used

14     Sprite 4 (or 5) to Sprite 6 (or 7)

13     Sprite 2 (or 3) to Sprite 6 (or 7)

12     Sprite 2 (or 3) to Sprite 4 (or 5)

11     Sprite 0 (or 1) to Sprite 6 (or 7)

10     Sprite 0 (or 1) to Sprite 4 (or 5)

09     Sprite 0 (or 1) to Sprite 2 (or 3)

08     Playfield 2 to Sprite 6 (or 7)

07     Playfield 2 to Sprite 4 (or 5)

06     Playfield 2 to Sprite 2 (or 3)

05     Playfield 2 to Sprite 0 (or 1)

04     Playfield 1 to Sprite 6 (or 7)

03     Playfield 1 to Sprite 4 (or 5)

02     Playfield 1 to Sprite 2 (or 3)

01     Playfield 1 to Sprite 0 (or 1)

00     Playfield 2 to Playfield 2

COLORxx  180-1BE  W             COLOR table xx

There 32 of these registers (xx=00-31) and together

with the banking bits they address the 256 locations in

the color palette.

There are actually two sets of color regs, selection of

which is controlled by the LOCT reg bit. When LOCT = 0

the 4 MSB of red, green and blue video data are

selected along with the T bit for genlocks the low

order set of registers is also selected as well, so

that the 4 bi valuesare automatically extended to 8

bits.This provides compatibility with old software. If

the full range of palette values are desired, then LOCT

can be set high and independant values for the 4 LSB of

red, green  and blue can be written. The low order

color registers do not contain a transparency (T) bit.

The table below shows the color register bit usage.

BIT#    15,14,13,12    11,10,09,08    07,06,05,04    03,02,01,00

----    -----------    -----------    -----------    -----------

LOCT=0  T  X  X  X     R7 R6 R5 R4    G7 G6 G5 G4    B7 B6 B5 B4

LOCT=1  X  X  X  X     R3 R2 R1 R0    G3 G2 G1 G0    B3 B2 B1 B0

T = TRANSPARENCY      R = RED    G = GREEN    B = BLUE    X = UNUSED

T bit of COLOR00 thru COLOR31 sets ZD_pin HI, When that

color is selected in all video modes.

COPCON h  02E  W     A          Coproccessor control register

This is a-1 bit register that when set true, allows

the coprocessor to access the blitter hardware this bit

is cleared power on reset, so that the coprocessor

cannot access the blitter hardware.

BIT# NAME   FUNCTION

---- ----   --------

01   CDANG  Coprocessor danger mode. Allows coprocessor

access to all RGA registers if true.

(if 0, access to RGA>7E)

(On old chips access to only RGA>3E if CDANG=1)

(see VPOSR)

COPJMP1   088  S    A           Coprocessor restart at first location

COPJMP2   08A  S    A           Coprocessor restart at second location

These address are strobe address, that when written to

cause the coprocessor to jump indirect useing the add-

ress contained in the first or second location regs

described below. The coprocessor itself can write to

these address, causeing it`s own jump indirect.

COP1LCH h 080  W    A         A Coprocessor first location reg

(high 5 bits) (old-3 bits)

COP1LCL   082  W    A         A Coprocessor first location reg

(low 15 bits)

COP2LCH h 084  W    A         A Coprocessor second location reg

(high 5 bits) (old-3 bits)

COP2LCL   086  W    A         A Coprocessor second location reg

(low 15 bits)

These regs contain the jump addresses described below

COPINS    08C  W   A            Coprocessor inst. fetch identify

This is a dummy address that is generated by the co-

processor whenever it is loading instructions into its

own instruction register.

This actually occurs every coprocessor cycle except

for the second (IR2) cycle of the MOVE instruction. The

three types of instructions are shown below.

MOVE Move immediate to dest

WAIT Wait until beam counter is equal to, or greater

than. (Keeps coprocessor off of bus until beam position

has been reached)

SKIP Skip if beam counter is equal to, or greater than.

(skips following MOVE inst. unless beam position has

been reached)

MOVE             WAIT UNTIL               SKIP IF

------------------------------------------------------

BIT#     IR1        IR2       IR1       IR2       IR1       IR2

----     ---        ---       ---       ---       ---       ---

15        x         RD15      VP7       BFD       VP7       BFD

14        x         RD14      VP6       VE6       VP6       VE6

13        x         RD13      VP5       VE5       VP5       VE5

12	x	RD12	VP4	VE4	VP4	VE4

11	x	RD11	VP3	VE3	VP3	VE3

10	x	RD10	VP2	VE2	VP2	VE2

09	x	RD09	VP1	VE1	VP1	VE1

08       DA8	RD08	VP0	VE0	VP0	VE0

07       DA7	RD07	HP8	HE8	HP8	HE8

06       DA6	RD06	HP7	HE7	HP7	HE7

05       DA5	RD05	HP6	HE6	HP6	HE6

04       DA4	RD04	HP5	HE5	HP5	HE5

03       DA3	RD03	HP4	HE4	HP4	HE4

02       DA2	RD02	HP3	HE3	HP3	HE3

01       DA1	RD01	HP2	HE2	HP2	HE2

00        0	RD00	 1	 0	 1	 1

IR1=First instruction register

IR2=Second insturction register

DA =Destination address for MOVE instruction.Fetched during

IR1 time,used during IR2 time on RGA bus.

RD =RAM Data moved by MOVE instruction at IR2 time

directly from RAM to the address given by the DA field.

VP =Vertical beam position comparison bit.

HP =Horizontal beam position comparison bit.

VE =Enable comparison (mask bit)

HE -Enable comparison (mask bit)

* NOTE BFD=Blitter finished disable. When this bit is true,

the blitter finished flag will have no

effect on the coprocessor. When this bit is zero

the blitter finished flag must be true

(in addition to the rest of the bit comparisons)

before the coprocessor can exit from it`s wait

state, or skip over an instruction. Note  that the

V7 comparison cannot be masked.

The coprocessor is basically a 2 cycle machine that requests

the bus only during odd memory cycles. (4 memory cycles per in)

It has priority over the blitter and micro.

There are only three types of instructions, MOVE immediate,

WAIT until ,and SKIP if. All instructions require 2 bus cycles

(and two instruction words).Since only the odd bus cycles are

requested, 4 memory cycle times are required per instruction.

(memory cycles are 280 ns)

There are two indirect jump registers COP1LC and COP2LC.

These are 20 bit pointer registers whose contents are

used to modify program counter for initalization or jumps.

They are transfered to the program counter whenever strobe

address COPJMP1 or COPJMP2 are written.In addition  COP1LC

is automatically used at the beginning of each vertical

blank time.

It is important that one of the jump registers be initalized

and it`s jump strobe address hit, after power up but before

coprocessor DMAis initalized.T his insures a determined

startup address, and state.

DDFSTRT  092  W  A  Display data fetch start(horiz. position)

DDFSTOP  094  W  A  Display data fetch stop (horiz. position)

These registers control the horizontal timing of the beginning

and end of the bit plane DMA timing display data fetch.

The vertical bit plane DMA timing is identical to the display

windows described above.

The bit plane Modulos are dependent on the bit plane

horizontal size, and on this data fetch window size.

Register bit assignment

-----------------------

BIT# 15  14  13  12  11  10  09  08  07  06  05  04  03  02  01  00

USE  XX   X   X   X   X   X   X  H8  H7  H6  H5  H4  H3  H2   X

(X bits should always be driven with 0 to maintain upward

compatability)

The tables below show the start and stop timing for

different register contents

DDFSTRT (Left edge of display data fetch)

------------------------------------------

PURPOSE 	   H8  H7  H6  H5  H4

------------------------------------------

Extra wide (max)    0   0   1   0   1

wide	    0   0   1   1   0

normal	    0   0   1   1   1

narrow	    0   1   0   0   0

DDFSTOP (Right edge of display data fetch)

------------------------------------------

------------------------------------------

PURPOSE 	   H8  H7  H6  H5  H4

------------------------------------------

narrow	    1   1   0   0   1

normal	    1   1   0   1   0

wide (max)	    1   1   0   1   1

Note that these numbers will vary with variable beam counter

mode set: (The maxes and mins, that is)

DIWSTRT 08E  W  A D   Display window start (upper left vert-hor pos)

DIWSTOP 090  W  A D   Display window stop (lower right vert-hor pos)

These registers control the display window size and position,

by locating the upper left and lower right corners.

BIT# 15  14  13  12  11  10  09  08  07  06  05  04  03  02  01  00

USE  V7  V6  V5  V4  V3  V2  V1  V0  H9  H8  H7  H6  H5  H4  H3  H2

DIWSTRT is vertically restricted to the upper 2/3

of the display (v8=0),and horizontally restricted to the

left 3/4 of the display (H8=0).*

* Poof.. (see DIWHIGH for exceptions) don`t ask me i didn`t write it?

DIWHIGH  p 1E4  W  A D Display window upper bits for start, stop

this is an added register for Hires chips, and allows

larger start & stop ranges. If it is not written, the above

(DIWSTRT,STOP) description holds. If this register is written,

direct start & stop positions anywhere on the screen.

It doesn`t affect the UHRES pointers.

BIT# 15  14  13  12  11  10  09  08  07  06  05  04  03  02  01  00

X   X  H10 H1  H0  V10 V9  V8   X   X  H10 H1  H0  V10 V9  V8

(stop)                 |         (start)

Take care (X) bits should always be written to 0 to

maintain upwards compatibility. H1 and H0 values define

70ns amd 35ns increments respectively, and new LISA bits.

NOTE: In all 3 display window registers, horizontal bit

positions have been renamed to reflect HIRES pixel

increments, e.g. what used to be called H0 is now referred to as H2.

DMACON  096  W  A D P DMA control write (clear or set)

DMACONR 002  R  A   P DMA control (and blitter status) read

This register controls all of the DMA channels, and contains

blitter DMA status bits.

BIT#    FUNCTION       DESCRIPTION

----    --------   ------------------------------------------------

15     SET/CLR	   Set/Clear control bit. Determines if bits

written wit a 1 get set or cleared.

Bits written witn a zero are unchanged.

14     BBUSY	   Blitter busy status bit (read only)

13     BZERO          Blitter logic zero status bit. (read only)

12     X

11     X

10     BLTPRI	   Blitter DMA prioiry (over CPU micro)

(also called "blitter nasty")

(disables /BLS pin, preventing micro

from stealing any bus cycles while

blitter DMA is running)

09     DMAEN	   Enable all DMA below (also UHRES DMA)

08     BPLEN          Bit plane DMA enable

07     COPEN	   Coprocessor DMA enable

06     BLTEN	   Blitter DMA enable

05     SPREN	   Sprite DMA enable

04     DSKEN	   Disk DMA enable

03     AUD3EN         Audio chanel 3 DMA enable

02     AUD2EN         Audio chanel 2 DMA enable

01     AUD1EN         Audio chanel 1 DMA enable

00     AUD0EN         Audio chanel 0 DMA enable

DSKPTH h 020 W  A	Disk pointer (high 5 bits) (old-3 bits)

DSKPTL   022 W  A	Disk pointer (low 15 bits)

This pair of registers contains the 20 bit address

of disk DMA data. These address registers must be

initalized by the processor or coprocessor

before disk DMA is enabled.

DSKLEN	024 W		P    Disk length

This register contains the length (number of words) of

disk DMA data. It also contains 2 control bits. These

are a DMA enable bit, and a DMA direction (read/write) bit.

BIT#    FUNCTION   DESCRIPTION

----    --------   ------------------------------------------------

15	DMAEN	   Disk DMA enable

14	WRITE      Disk write (RAM or disk) if 1

13-0   LENGTH     LENGTH (# of words) of DMA data.

DSKDAT	026 W  P  Disk DMA data write

DSKDATR 008 ER   P  Disk DMA  data read (early read dummy address)

This register is the disk-DMA data buffer.It contains 2

bytes of data that are either sent to (write) or received

from (read) the disk. The DMA controller automatically

transfers data to or from this register and RAM, and when

the DMA data is finished (length=0) it causes a disk

block interrupt. See interrupts below.

DSKBYTR 01A  R  p Disk data byte and status read

This register is the Disk-Microrocessor data buffer.

Data from the disk (in read mode) is leaded into this

register one byte at a time, and bit 15 (DSKBYT) is set true.

BIT#    FUNCTION   DESCRIPTION

----    --------   ------------------------------------------------

15     DSKBYT     Disk byte ready (reset on read)

14     DMAON      DMAEN (DSKLEN) & DMAEN (DMACON) & DSKEN (DMACON)

13     DISKWRITE  Mirror of bit 14 (WRITE) in DSKLEN

12     WORDEQUAL  This bit true only while DSKSYNC register

equals the data from disk

11-08  0	         Not used

07-00  DATA       Disk byte data

DSKSYNC  07E W  	P Disk sync register,the match code for disk

read synchronization. See ADKCON bit 10

FMODE P  1FC W 		  Memory Fetch Mode

This register controls the fetch mechanism for different

types of Chip RAM accesses:

BIT#    FUNCTION       DESCRIPTION

----    --------       -------------------------------------------

15     SSCAN2       Global enable for sprite scan-doubling.

14     BSCAN2	   Enables the use of 2nd P/F modulus on an

alternate line basis to support bitplane

scan-doubling.

13-04  Unused

03     SPAGEM         Sprite page mode (double CAS)

02     SPR32        Sprite 32 bit wide mode

01     BPAGEM         Bitplane Page Mode (double CAS)

00     BLP32	   Bitplane 32 bit wide mode

BPAGEM BPL32 Bitplane Fetch Increment Memory Cycle Bus Width

------------------------------------------------------------

0	0    By 2 bytes    (as before) normal CAS     16

0	1    By 4 bytes 	       normal CAS     32

1	0    By 4 bytes 	       double CAS     16

1 	1    By 8 bytes      	       double CAS 32

SPAGEM SPR32 Sprite Fetch Increment  Memory Cycle  Bus Width

------------------------------------------------------------

0     0    By 2 bytes    (as before) normal CAS     16

0     1    By 4 bytes                normal CAS     32

1     0    By 4 bytes                double CAS     16

1     1    By 8 bytes                double CAS     32

HBSTOP 1C6 W  D    Horizontal STOP position

HBSTRT 1C4 W  D    Horizontal START position

Bits 7-0 contain the stop and start positions,

respectively, for programed horizontal blanking in

280nS increments.Bits 10-8 provide a fine position

control in 35nS increments.

BIT#    FUNCTION   DESCRIPTION

----    --------   ------------------------------------------------

15-11  x          (unused)

10     H1         140nS

09     H1         70nS

08     H0         35nS

07     H10        35840nS

06     H9         17920nS

05     H8         8960nS

04     H7         4480nS

03     H6         2240nS

02     H5         1120nS

01     H4         560nS

00     H3         280nS

HCENTER H 1E2  W  A  Horizontal position (CCKs) of VSYNC on long field

this is necessary for interlace mode with variable beam

counters. See BEAMCON0 for when it affects chip outputs.

See HTOTAL for bits.

HHPOSR  H 1DA  R  A  DUAL mode hires Hbeam counter read

HHPOSW  H 1D8  W  A  DUAL mode hires Hbeam counter write

This the secondary beam counter for

the faster mode, triggering the UHRES pointers & doing the

comparisons for HBSTRT,STOP,HTOTAL,HSSRT,HSSTOP

(See HTOTAL for bits)

HHSTOP H  1C2  W  A   Horiz line position for SYNC stop

Sets # of colour clocks for sync stop (HTOTAL for bits)

HHSTRT H  1DE  W  A  Horiz line position for HSYNC stop

Sets # of colour clocks for sync start (HTOTAL for bits)

See BEAMCON0 for details of when these 2 are active.

HTOTAL H  1C0  W  A    Highest colour clock count in horiz line

BIT#  15  14  13  12  11  10  09  08  07  06  05  04  03  02  01  00

x   x   x   x   x   x   x   x  h8  h7  h6  h5  h4  h3  h2  h1

(x`s should be driven to 0 for upward compatibility)

Horiz line has theis many + 1 280nS increments. If the

pal bit & LOLDIS are not high, long line/skort line toggle

will occur, and there will be this many +2 every other line.

Active if VARBEAMEN=1 or DUAL+1.

INTREQ   09C  W         P Interrupt request bits (clear or set)

INTREQR  01E  R         P Interrupt request bits (read)

This register contains interrupt request bits (or flags).

These bits may be polled by the processor, and if enabled

by the bits listed in the next register, they may cause

processor interrupts. Both a set and clear operation

are required to load arbitary data into this register.

The bit assignments are identical to the enable register

below.

INTENA   09A  W 	   P Interrupt enable bits (clear or set bits)

INTENAR  01C  R        P Interrupt enable bits (read)

This register contains interrupt enable bits. The bit

assignment for both the request, and enable registers

is given below.

BIT#    FUNCTION  LEVEL DESCRIPTION

----    --------  ----- ----------------------------------------

15     SET/CLR         Set/clear control bit. Determines if bits

written with a 1 get set or cleared. Bits

written with a zero are always unchanged.

14     INTEN	    Master interrupt (enable only, no request)

13     EXTER      6    External interrupt

12     DSKSYN     5    Disk sync register (DSKSYNC) matches disk

11     RBF        5    Serial port receive buffer full

10     AUD3       4    Audio channel 3 block finished

09     AUD2       4    Audio channel 2 block finished

08     AUD1       4    Audio channel 1 block finished

07     AUD0       4    Audio channel 0 block finished

06     BLIT       3    Blitter has finished

05     VERTB      3    Start of vertical blank

04     COPER      3    Coprocessor

03     PORTS      2    I/O Ports and timers

02     SOFT       1    Reserved for software initated interrupt.

01     DSKBLK     1    Disk block finished

00     TBE        1    Serial port transmit buffer empty

JOY0DAT  00A R     D   Joystick-mouse 0 data (left vert, horiz)

JOY1DAT  00C R     D   Joystick-mouse 1 data (right vert,horiz)

These addresses each read a 16 bit register. These in turn

are loaded from the MDAT serial stream and are clocked in on

the rising edge of SCLK. MLD output is used to parallel load

the external parallel-to-serial converter.This in turn is

loaded with the 4 quadrature inputs from each of two game

controller ports (8 total) plus 8 miscellaneous control bits

which are new for LISA and can be read in upper 8 bits of

LISAID.

Register bits are as follows:

Mouse counter usage (pins  1,3 =Yclock, pins 2,4 =Xclock)

BIT#  15  14  13  12  11  10  09  08     07  06  05  04  03  02  01  00

JOY0DAT   Y7  Y6  Y5  Y4  Y3  Y2  Y1  Y0     X7  X6  X5  X4  X3  X2  X1  X0

JOY1DAT   Y7  Y6  Y5  Y4  Y3  Y2  Y1  Y0     X7  X6  X5  X4  X3  X2  X1  X0

0=LEFT CONTROLLER PAIR, 1=RIGHT CONTROLLER PAIR.

(4 counters total).The bit usage for both left and right

addresses is shown below. Each 6 bit counter (Y7-Y2,X7-X2) is

clocked by 2 of the signals input from the mouse serial

stream. Starting with first bit recived:

Serial Bit Name	Description

--------------------------------------------------------------------

0    | M0H  | JOY0DAT Horizontal Clock

1    | M0HQ | JOY0DAT Horizontal Clock (quadrature)

2    | M0V  | JOY0DAT Vertical Clock

3    | M0VQ | JOY0DAT Vertical Clock  (quadrature)

4    | M1V  | JOY1DAT Horizontall Clock

5    | M1VQ | JOY1DAT Horizontall Clock (quadrature)

6    | M1V  | JOY1DAT Vertical Clock

7    | M1VQ | JOY1DAT Vertical Clock (quadrature)

Bits 1 and 0 of each counter (Y1-Y0,X1-X0) may be

read to determine the state of the related input signal pair.

This allows these pins to double as joystick switch inputs.

Joystick switch closures can be deciphered as follows:

Directions     Pin#     Counter bits

----------     ----	  --------------------

Forward          1      Y1 xor Y0 (BIT#09 xor BIT#08)

Left             3      Y1

Back             2      X1 xor X0 (BIT#01 xor BIT#00)

Right            4      X1

JOYTEST  036 W   D     Write to all 4  joystick-mouse counters at once.

Mouse counter write test data:

BIT#  15  14  13  12  11  10  09  08     07  06  05  04  03  02  01  00

JOY0DAT  Y7  Y6  Y5  Y4  Y3  Y2  xx  xx     X7  X6  X5  X4  X3  X2  xx  xx

JOY1DAT  Y7  Y6  Y5  Y4  Y3  Y2  xx  xx     X7  X6  X5  X4  X3  X2  xx  xx

LISAID  H  07C  R   D    Denise/Lisa (video out chip) revision level

The original Denise (8362) does not have this register, so

whatever value is left over on the bus from the last cycle

will be there. ECS Denise (8373) returns hex (fc) in the lower

8 bits.Lisa returns hex (f8). The upper 8 bits of this

Register are loaded from the serial mouse bus, and are

reserved for future hardware implentation.

The 8 low-order bits are encoded as follows:

BIT#  Description

----  --------------------------------------------------

7-4  Lisa/Denise/ECS Denise Revision level(decrement to

bump revision level, hex F represents 0th rev. level).

3  Maintain as a 1 for future generation

2  When low indicates AA feature set (LISA)

1  When low indicates ECS feature set (LISA or ECS DENISE)

0  Maintain as a 1 for future generation

POT0DAT h  012  R       P  Pot counter data left pair (vert, horiz)

POT1DAT h  014  R       P  Pot counter data right pair (vert,horiz)

These addresses each read a pair of 8 bit pot counters.

(4 counters total). The bit assignment for both

addresses is shown below. The counters are stopped by signals

from 2 controller connectors (left-right) with 2 pins each.

BIT#  15  14  13  12  11  10  09  08     07  06  05  04  03  02  01  00

RIGHT  Y7  Y6  Y5  Y4  Y3  Y2  Y1  Y0     X7  X6  X5  X4  X3  X2  X1  X0

LEFT  Y7  Y6  Y5  Y4  Y3  Y2  Y1  Y0     X7  X6  X5  X4  X3  X2  X1  X0

CONNECTORS

-------------------------PAULA

Loc.    dir.  sym   pin   pin

-----   ----- ----- ----- -----

RIGHT   Y     RX     9    33

RIGHT   X     RX     5    32

LEFT   Y     LY     9    36

LEFT   X     LX     5    35

With normal (NTSC or PAL) horiz. line rate, the pots will

give a full scale (FF) reading with about 500kohms in one

frame time. With proportionally faster horiz line times,

the counters will count proportionally faster.

This should be noted when doing variable beam displays.

POTGO  034 W      P   Pot port (4 bit) bi-direction and data,

and pot counter start.

POTINP 016 R      P Pot pin data read

This register controls a 4 bit bi-direction I/O port

that shares the same 4 pins as the 4 pot counters above.

BIT#    FUNCTION   DESCRIPTION

----    --------   ----------------------------------------

15     OUTRY      Output enable for Paula pin 33

14     DATRY      I/O data Paula pin 33

13     OUTRX      Output enable for Paula pin 32

12     DATRX      I/O data Paula pin 32

11     OUTLY      Out put enable for Paula pin 36

10     DATLY      I/O data Paula pin 36

09     OUTLX      Output enable for Paula pin 35

08     DATLX      I/O data  Paula pin 35

07-01    X        Not used

00     START      Start pots (dump capacitors,start counters)

REFPTR   028  W   A     Refresh pointer

This register is used as a dynamic RAM refresh address

generator. It is writeable for test purposes only,and

should never be written by the microprocesor.

SERDAT  030  W      P   Serial port data and stop bits write.

This address writes data to a transmit data buffer.

Data from this buffer is moved into a serial shift register

for output transmission whenever it is empty.This sets the

interrupt request TBE  (transmit buffer empty). A stop bit

must be provided as part of the data word. The length

of the data word is set by the position of the stop bit.

BIT#  15  14  13  12  11  10  09  08     07  06  05  04  03  02  01  00

USE    0   0   0   0   0   0   S  D8     D7  D6  D5  D4  D3  D2  D1  D0

Note :  S= Stop bit =1, D= data bits

SERDATR  018  R      P  Serial port data and status read.

This address reads data from a recive data buffer.

Data in this buffer is loaded from a receiving shift

register whenever it is full. Several interrupt request

bits are also read at this address, along with the data as

shown below.

BIT#    FUNCTION   DESCRIPTION

----    --------   ----------------------------------------

15     OVRUN      Serial port receiver overun

14     RBF        Serial port receive buffer full (mirror)

13     TBE        Serial port transmit buffer empty (mirror)

12     TSRE       Serial port transmit shift reg. empty

11     RXD        RXD pin receives UART serial data for

direct bit test by the micro.

10     X          Not used.

09     STP        Stop bit

08   STP-DB8      Stop bit if LONG, data bit if not.

07     DB7        Data bit.

06     DB6        Data bit.

05     DB5        Data bit.

04     DB4        Data bit.

03     DB3        Data bit.

02     DB2        Data bit.

01     DB1        Data bit.

00     DB0        Data bit.

SERPER  032  W        P    Serial port period and control.

This register contains the control bit LONG reffered to

above, and a 15 bit number defining the serial port

Baud rate.If this number is N,then the baud rate is

1 bit every (N+1)*.2794 microseconds.

BIT#    FUNCTION   DESCRIPTION

----    --------   ----------------------------------------

15     LONG       Defines serial receive as 9 bit word.

14-00  RATE       Defines baud rate=1/((N+1)*.2794 microseconds)

SPRHDAT  H 078   W  exe logic UHRES sprite identifier and data

This identifies the cycle when this pointer address is on

the bus accessing the memory.

SPRHPTH  H  1E8  W   A    UHRES sprite pointer (high 5 bits)

SPRHPTL  H  1EA  W   A    UHRES sprite pointer (low 15 bits)

This pointer is activated in the 1st and 3rd `free` cycles

(see BPLHPTH,L) after horiz line start.It increments

for the next line.

SPRHSTOP H 1D2  W    A    UHRES sprite vertical display stop

BIT#  15  14  13  12  11  10  09  08     07  06  05  04  03  02  01  00

SPRHWRM   x   x   x   x   x  v10  v9  v8     v7  v6  v5  v4  v3  v2  v1  v0

SPRHWRM = Swaps the polarity of ARW* when the SPRHDAT comes

out so that external devices can detect the RGA and put

things into memory.(ECS and later chips only)

SPRHSTRT H 1D0  W   A     UHRES sprite vertical display start

BIT#  15  14  13  12  11  10  09  08     07  06  05  04  03  02  01  00

x   x   x   x   x  v10 v9  v8     v7  v6  v5  v4  v3  v2  v1  v0

SPRxPTH    120  W    A      Sprite x pointer (High 5 bits)

SPRxPTL    122  W    A      Sprite x pointer (low 15 bits)

This pair of registers contains the 20 bit address

of sprite x (x=0,1,2,3,4,5,6,7) DMA data.These address

registers must be initalized by the processor or coprocessor

every vertical blank time.

SPRxPOS   140   W    A  D   Sprite x vert-horiz start position data.

BIT#	SYM		FUNCTION

----    ----    	-----------------------------------------

15-08  SV7-SV0  	Start vertical value.High bit (SV8) is

in SPRxCTL register below.

07-00  SH10-SH3 	Sprite horizontal start value. Low order

3 bits are in SPRxCTL register below. If

SSCAN2 bit in FMODE is set, then disable

SH10 horizontal coincidence detect.This bit

is then free to be used by ALICE as an

individual scan double enable.

SPRxCTL p 142 W     A  D   Sprite position and control data

BIT#	SYM		FUNCTION

----   ----    	----------------------------------------

15-08  EV7-EV0         End (stop) vert. value. Low 8 bits

07    ATT	    Sprite attach control bit (odd sprites only)

06    SV9	    Start vert value 10th bit.

05    EV9             End (stop) vert. value 10th bit

04   SH1=0            Start horiz. value, 70nS increment

03   SH0=0            Start horiz. value 35nS increment

02    SV8             Start vert. value 9th bit

01    EV8             End (stop) vert. value 9th bit

00    SH2             Start horiz.value,140nS increment

These 2 registers work together as position, size and

feature sprite control registers.They are usually loaded

by the sprite DMA channel, during horizontal blank,

however they may be loaded by either processor any time.

Writing to SPRxCTL disables the corresponding sprite.

SPRxDATA  144  W      D    Sprite x image data register A

SPRxDATB  146  W      D    Sprite x image data register B

These registers buffer the sprite image data.They are

usually loaded by the sprite DMA channel but may be

loaded by either processor at any time. When a horizontal

coincidence occurs the buffers are dumped into shift

registers and serially outputed to the display, MSB first

on the left.

NOTE: Writing to the A buffer enables (arms) the sprite.

Writing to the SPRxCTL registers disables the sprite.

If enabled, data in the A and B buffers will be output

whenever the beam counter equals the sprite horizontal

position value in the SPRxPOS register. In lowres mode,

1 sprite pixel is 1 bitplane pixel wide.In HRES and

SHRES mode, 1 sprite pixel is 2 bitplane pixels.

The DATB bits are the 2SBs (worth 2) for the color registers,

and MSB for SHRES. DATA bits are LSBs of the pixels.

STREQU    038   S    D    Strobe for horiz sync with VB (vert blank) and EQU

STRVBL    038   S    D    Strobe for horiz sync with VB

STRHOR    03C   S    D P  Strobe for horiz sync

STRLONG h 03E   S    D    Strobe for identification of long horiz line (228CC)

One of the first 3 strobe addresses above, it is

placed on the RGA bus during the first refresh time

slot of every other line, to identify lines with

long counts (228- NTSC, HTOTAL+2- VARBEAMEN=1

hires chips only).There are 4 refresh time slots

and any not used for strobes will leave a null

(1FE) address on the RGA bus.

VBSTOP H 1CE   W    A    Vertical line for VBLANK stop

VBSRTR H 1CC   W    A    Vertical line for VBLANK start

(V10-0 <- D10-0) Affects CSY pin if BLAKEN=1 and VSY pin

if CSCBEN=1 (see BEAMCON0)

VPOSR  p 004   R    A   Read vert most sig. bits (and frame flop)

VPOSW    02A   W    A   Write most sig. bits (and frame flop)

BIT#  15  14  13  12  11  10  09  08     07  06  05  04  03  02  01  00

USE LOF   I6  I5  I4  I3  I2  I1  I0 LOL --  --  --  --  --  V10 V9  V8

LOF=Long frame(auto toggle control bit in BPLCON0)

I0-I6 Chip identitication:

8361 (Regular) or 8370 (Fat) (Agnus-ntsc)=10

8367 (Pal) or 8371 (Fat-Pal) (Agnus-pal)=00

8372 (Fat-hr) (agnushr),thru rev4 = 20 Pal,30 NTSC

8372 (Fat-hr) (agnushr),rev 5 = 22 Pal, 31 NTSC

8374 (Alice) thru rev 2 = 22 Pal, 32 NTSC

8374 (Alice) rev 3 thru rev 4 = 23 Pal, 33 NTSC

LOL = Long line bit. When low, it indicates short raster

line.

v9,10 -- hires chips only (20,30 identifiers)

VHPOSR     006  R   A    Read vert and horiz position of beam, or lightpen

VHPOSW     02C  W   A    Write vert horiz position of beam, or lightpen

BIT#  15  14  13  12  11  10  09  08     07  06  05  04  03  02  01  00

USE   V7  V6  V5  V4  V3  V2  V1  V0     H8  H7  H6  H5  H4  H3  H2  H1

RESOLUTION = 1/160 of SCREEN WITH ( 280 nS)

VSSTOP H   1CA W   A   Vert position for VSYNC start

VTOTAL H   1C8 W   A   Highest numbered vertival line (VARBEAMEN = 1)

It`s the line number to reset the counter,

so there`s this many + 1 in a field. The exception is

if the LACE bit is set (BPLCON0), in which case every

other field is this many + 2 and the short field is this

many + 1.

5.    NEW LISA DISPLAY MODES

We now have a palette of 2~24 colours.

LORES  (320x200)

----------------

6 Bitplane (non HAM, non EHB) 64 colours                    !

7 Bitplane 		128 colours		!

8 Bitplane		256 colours		!

8 Bitplane HAM  	Any 2~24 colours                        !

Dual playfield, Max 4 bitplane per playfield                !

16 colours per playfield.The bank of 16 colours

in the 256 colour palette is selectabel per playfield

HIRES  (640x200)

----------------

5 Bitplanes		32 colours		@

6 Bitplanes		64 colours		@

7 Bitplanes		128 colours		@

8 Bitplanes		256 colours		@

6 Bitplanes EHB  	32 * 2 colours			@

6 Bitplanes HAM		4096 colours		@

8 Bitplanes HAM		any of 2~256 colours	@

Dual playfield, max 4 bitplane per playfield	        ! or @

16 colours per playfield. The bank of 16 colours

in the 256 colour palette is selectable per playfield.

SUPERHIRES (1280X200)

---------------------

1 or 2 bitplanes, as ECS, but no colour fudging		!

3 Bitplanes		8 colours			@

4 Bitplanes		16 colours		@

5 Bitplanes		32 colours		$

6 Bitplanes		64 colours		$

7 Bitplanes		128 colours		$

8 Bitplanes		256 colours		$

6 Bitplanes EHB         32 * 2 colours			$

6 Bitplanes HAM		4O96 colours		$

8 Bitplanes HAM         any of 2~24 colours		$

Dual Playfield, max 4 bitplanes per playfield   @ or $

16 colours per playfield. The bank of 16 of colours

in the 256 colours palette is selectable per playfield.

VGA  (640X480 non-interlaced)

-----------------------------

1 or 2 bitplanes, as ECS, but no colour fudging		!

3 Bitplanes		8 colours			@

4 Bitplanes		16 colours		@

5 Bitplanes		32 colours		$

6 Bitplanes		64 colours		$

7 Bitplanes		128 colours		$

8 Bitplanes		256 colours		$

6 Bitplanes EHB         32 * 2 colours			$

6 Bitplanes HAM		4O96 colours		$

8 Bitplanes HAM         any of 2~24 colours		$

Dual playfield,Max 4 bitplanes per playfield	        @ or $

16 colours per playfield . The bank of 16 colours

in the 256 colour palette is selectable per playfield

Super 72 (848x614 interlaced, 70 Hz frame rate)

-----------------------------------------------

1 or 2 bitplanes, as ECS, but no colour fudging		1X

3 Bitplanes		8 colours			2X

4 Bitplanes		16 colours		2X

5 Bitplanes		32 colours		4X

6 Bitplanes		64 colours		4X

7 Bitplanes		128 colours		4X

8 Bitplanes		256 colours		4X

6 Bitplanes EHB               32 * 2 colours		4X

6 Bitplanes HAM		4O96 colours		4X

8 Bitplanes HAM         any of 2~24 colours		4X

Dual playfield,Max 4 bitplanes per playfield	        2X or 4X

16 colours per playfield . The bank of 16 colours

in the 256 colour palette is selectable per playfield

All playfield scrolling is now in 35ns increments.

Pre AA scrolling was in 140ns increments.

SCROLL RANGE AS PROGRAMMED IN BPLCON1

-------------------------------------

COMBAT 18 ARE LOOKING FOR COOL CODERS,TRADERS,ECT..

=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*

������� CALL OUR BOARDS WORLDWIDE ������

=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*=*

COMBAT 18 Greets LSD,FAIRLIGHT,SILENTS,DUAL CREW,SKID ROW

Rose